HOLLOO Technology

Departmental Standards Agreement

# -V JT . 04 .088

Shenzhen Chehulu Technology Co., Ltd.

| Serial number | Version Number | Revisions | Revised by | Revision Date |
| --- | --- | --- | --- | --- |
| 1 | V1.01.000 | First Draft | XQM | 2017-8-24​​​ |
| 2 | V 1.01.001 | Content proofreading and formatting | WDC | 2018- 9-6 |
| 3 | V 1.01.00 2 | 1.0x0200 Extended information added Manufacturer-defined ID "0 x FE" extension , Sub-extension items 0x01, 0x02 extend GSM/CDMA base station information . | YD | 2018-9-12 |
| 4 | V1.01.003 | Update the old version 1.04 protocol content | WDC | 2018-9-19 |
| 5 | VJT.01.001 | Re-editing protocol: - Fault code data, travel data, sleep data, wake-up data are transparently transmitted through 0900. - The settings for rapid acceleration, rapid deceleration, and rapid turning are changed to 4 levels: high, medium, low, and off. - Add version information package, the terminal reports software version/VIN code and other information, and the platform responds with system time, making it easier to calibrate the terminal time. - 0200 data is still packaged and reported together in a dynamic manner. - Added the function of setting privilege number - Added escape and unescaping function examples | XQM | 2018-11-19 |
| 6 | VJT.01.002 | - Add longitude and latitude to the fault code data packet. - Add longitude and latitude to trip data - Adjust the basic data stream part of 0200 extended data to add CSQ and backup power voltage | XQM | 2018-12-10 |
| 7 | VJT.01.003 | 1. Add the latitude and longitude at the time of engine shutdown to driving behavior data | XQM | 2018-12-12 |
| 8 | VJT.01.004 | - Remove the number of packet inflection points in the line settings - Add inflection point ID, synchronized with the standard departmental protocol | XQM | 2018-12-18 |
| 9 | VJT.01.005 | 1. Version information ID uses 0205/8205 | XQM | 2018-12-20 |
| 10 | VJT.01.006 | 1. Some alarm data of 0200 are reported using independent extension ID 0xFA 2. In the 0200 extended data section, the command ID has been readjusted to be compatible with the command ID of previous customers 3. The 0900 F1 trip data package is changed to a dynamic data package. | XQM | 2019-01-04 |
| 11 | VJT.01.007 | 1. Details adjustment: set mileage, set three emergency collisions, expand some speeding alarms, water temperature alarms, expand command ID, accelerometer | XQM | 2019-01-08 |
| 12 | VJT.01.008 | - Some individual typos - 0200 basic extended data items, increase the number of satellites received, position accuracy, signal-to-noise ratio | XQM | 2019-01-09 |
| 12 | VJT.01.009 | - Modify the redundant package parameter items in the setting area configuration parameters - In the setting parameter 8103, add 0090 to configure the positioning mode field | XQM | 2019-01-15 |
| 1 3 | VJT.01.0 10 | 1. Add 0x6006 text message reply command | YGL | 2 019-01-21 |
| 14 | VJT.04.011 | 1. Added hyperlink function and unified version number starting from VJT.04.011 2. Modify the 0201 message body and add the serial number data item 3. Modify the 0200 message body and delete the position report message body length data item | LY | 2019-02-15 |
| 15 | VJT.04.012 | 1.0200 Add 0x001B GPS antenna status to the basic data stream of the extended data 0x001 C time calibration status to the basic data stream in the extended data 0x3008 H600 video status to the peripheral data stream in the extended data | YD | 2019-02-21 |
| 16 | VJT.04.013 | 1. 0x6210 fault mileage changed from 2 bytes to 4 bytes 2. 0x6110 Absolute throttle position changed from 1 byte to 2 bytes 3. 0x6070 Long-term fuel correction (cylinder banks 1 and 3) changed from 1 byte to 2 bytes 4. 0x60E0 The first cylinder ignition timing advance angle is changed from 1 byte to 2 bytes | LDY | 2019-02-26 |
| 17 | VJT.04.014 | 1.8103 Setting 8104 Query adds the following items: 0x2012: Set mileage, fuel consumption type 0x2013: Set mileage factor 0x2014: Set fuel consumption coefficient 0x2015: Set oil density 0x2016: Set idle fuel consumption coefficient 2.8205 The platform time is changed to Beijing Time Zone 8. 3. New extension 0x3009 H600 input signal quantity | YD | 2019-03-28 |
| 18 | VJT.04.015 | 1. Added truck extension data, compatible with some data streams of 32960 national standard | XQM | 2019-04-08 |
| 19 | VJT.04.016 | - Modify the problem that there is no length in front of each packet in 0704 packet data - The command word description in control command 8105 is incorrect | XQM | 2019-04-22 |
| 20 | VJT.04.017 | 1.0200 In the extended data, the public basic data item adds sub-ID 0x001D: positioning flag 2. Add 8103 sub-id 0x2017 to enable and disable OBD commands | XQM | 2019-04-23 |
| twenty one | VJT.04.018 | 1.0200 Extended data items, add truck data item sub-ID 0xFFF1 (mileage data) 0xFFF2 (fuel consumption data) 2.Configure query command 8103/8104, sub-function ID 0x2018 adds location data return mode, supports two types, one is first-in-first-out, the other is real-time data priority sending | XQM | 2019-05-23 |
| twenty two | VJT.04.019 | 1.0200 Add maintenance mode status to the status data item in the data 2. In the 0200 data, in the extended data item, the basic data item adds ID: 0x001E, which indicates the accumulated mileage. 3. In the 0900 data, F1 trip data, add a 0x001D to represent idle fuel consumption. | XQM | 2019-07-05 |
| twenty three | VJT.04.020 | 1.0200, in the status flag, bit 14 indicates the WIFI status, 1 is enabled; 0 is disabled. 2.WIFI parameter settings in 8103. 3.8103 sleep wake-up time setting. | XQM | 2019-08-12 |
| twenty four | VJT.04.021 | 1.0108 The upgrade result type adds the following item, 0xA2: GSM module 2. In 8105 remote control, add a type 0xF1, which means starting the GSM module OTA upgrade | XQM | 2019-08-27 |
| 25 | VJT.04.025 | 1. Delete all useless protocols, unify the font format, and align the upper and lower hyperlinks. | JJ | 2019-9-26 |
| 26 | VJT.04.026 | 1.0200 Added weighing sensor data to the extended peripheral data 2.0200 alarm extension data, add emergency brake, emergency brake, overspeed, PTO idle alarm, and 8103 configuration alarm parameters 3.0200 In the extended peripheral data, update the tire pressure data 4.0900 Added MCU upgrade status feedback package | XQM | 2019-9-30 |
| 27 | VJT.04.027 | 1.0200 In the tire data, high pressure, low pressure and high temperature status bits are added, and additional text is added to the tire temperature data description 2.8103 setting parameters conflict, readjust | XQM | 2019-10-21 |
| 28 | VJT.04.028 | 1.8103 configuration parameters are adjusted, 2012 command ID, add 0x0d pulse speed calculation mileage type 2.0200 Add instantaneous fuel consumption 3.0702 Add the original departmental standard function 4.8103 Sub-ID 0x2024 Flame-out delay time, supports setting and querying | XQM | 2019-11-15 |
| 2 9 | VJT.04.02 9 | 1. Truck data item 6 0B0 changes the data length to 2 bytes. 2. Delete two duplicate definitions of truck data item 510 B, 5110. 3. Truck data items refer to GB 17691-2018 , and new truck environmental protection related data items 5111-5118 are added | Y​ | 2 019-12-14 |
| 30 | VJT.04.0 30 | 1.0200 Data Truck Extended Data, intake pipe pressure, originally used 60B0 one byte, range is 1-255KPA, now without changing the original, use a new ID 50B0 two bytes, range is 1-500KPA. | XQM | 2 019-12- 20 |
| 31 | VJT.04.0 31 | 1.0200 Extended data items for cars, add 0x63C0 command ID, catalyst temperature 2.8300 text information, add a HUD text data. | XQM | 2 020-02- 26 |
| 32 | VJT.04.0 32 | 1. Add vehicle control commands, mainly expand the 8105 terminal control commands, and add a new 0105 control result response command 2. Added 8103 setting command, setting of parameters such as whether the ACC line is valid, fuel consumption coefficient, minimum interval of OBD speed data stream, etc. 3.0900 In the sleep exit data package, the sleep wake-up type is adjusted 4. Three new parameters added to the 0200 truck extended data: light absorption coefficient/opacity/particle concentration | XQM | 2 020-04- 27 |
| 33 | VJT.04.0 3 3 | 1. In the 0200 data basic data stream, add an ignition type command ID 0x0020, see the protocol for details 2. In the 0200 data basic data stream, the safety status in the 0x0011 vehicle status table adds a status bit, the engine status bit, see the protocol for details. | XQM | 2 020-0 8 - 06 |
| 34 | VJT.04.0 3 4 | Correction of basic data flow | TQL | 2 020-0 9 - 02 |
| 35 | VJT.04.0 3 5 | 1. Add department ID: 8202 temporary location tracking data package | XQM | 2 020-0 9 - 04 |
| 36 | VJT.04.0 3 6 | 1. Newly added 0200 extended data stream, 0XFB base station data packet. 2. Newly added 0200 car data, tire pressure, oil level, maintenance mileage, and number of collisions. | XQM | 2 020- 10 - 23 |
| 37 | VJT.04.0 3 7 | - Newly added 0200 extended data stream, basic data packet 0XEA, and added cumulative carbon emissions - Newly added 0200 extended data stream, truck data stream 0XEC, added engine current load 0x511F | XQM | 2020-12-29 |
| 38 | VJT.04.0 3 8 | 1. Newly added 0200 truck data stream 0XEC and powder loosening machine related data stream. | XQM | 2021-04-08 |
| 39 | VJT.04.0 3 9 | - Added 0200 truck data stream 0XEC, added powder loosening machine engine total running time 0x520A - Newly added 0200 basic data stream 0XEA, Roll angular velocity, Pitch angular velocity, Yaw angular velocity - New energy vehicle data, 0200 new energy data 0XED, data content adjustment, project type | XQM | 2021-05-17 |
| 40 | VJT.04.040 | 1. In the extended peripheral data 0xEE in the 0200 position data, add 0x300B external oil rod data stream, see the protocol details for details | XQM | 2021-09-10 |
| 41 | VJT.04.041 | 1. In the extended car data 0XEB in the 0200 position data, two extended sub-IDs are added to report AEB CAN message data | XQM | 2021-09-29 |
| 42 | VJT.04.042 | 1. Newly added 020A command to report the collected CAN data stream and customize the function 2. Add a differential pressure oil sensor to the 0200 data | XQM | 2022-01-17 |
| 43 | VJT.04.043 | - The current speed of the drive motor, add an offset of -32767 to distinguish between forward and reverse rotation. 2. 0x0105, control command response, plus the response serial number. 3. 0x0001 general response, new status: the previous instruction is in progress. | PZQ | 2022-09-20 |
| 44 | VJT.04.044 | 1. 0200 FA adds 0x0405~0x0408 alarms. 2. Adjust the alarm description of 0200 FA 0x0103~0x0104. | PZQ | 2022-10-11 |
| 45 | VJT.04.045 | 1. Add 0105 control command and failure reason. | PZQ | 2022-10-26 |
| 46 | VJT.04.046 | 0x00 25 to the basic data stream part of the extended data Accumulated mileage 2 (customized by SWD, used to calculate tire life) | LHL | 2022-10-27 |
| 47 | VJT.04.047 | 1. Add 6 channels to collect the relevant liquid position of fire trucks to expand the peripheral data flow | PZQ | 2022-11-04 |
| 48 | VJT.04.048 | 1. 0200 FA adds 0x040A low battery alarm | LHL | 2023-1-6 |
| 49 | VJT.04.049 | Add 0x520B--0x520E data items | XWB | 2023-1-10 |
| 50 | VJT.04.50 | 1. New energy vehicle data flow increases battery temperature | LHL | 2023-1-14 |
| 51 | VJT.04.51 | Expanded peripheral data streams to include OX300D and temperature sensor data streams | XWB | 2023-2-20 |
| 52 | VJT.04.52 | 1. Add extension ID 0xFC in 0x0200 extension information, wifi data stream | LHL | 2023-3-11 |
| 53 | VJT.04.53 | 1. Add ACC interrupt ignition type to the ignition type in the basic data stream of 0x0200. | PZQ | 2023-5-23 |
| 54 | VJT.04.54 | 1. The 13th bit in the alarm status bit of 0x0200 is supplemented with the overspeed warning flag bit. | PZQ | 2023-5-30 |
| 55 | VJT.04.55 | 1. Three new parameters are added to 0x8103 and 0x8104: 0x2029, 0x202A, 0x202B, Bluetooth authentication code, Bluetooth name, and Bluetooth MAC address. | PZQ | 2023-6-01 |
| 56 | VJT.04.56 | Added OBD1 (6X14), OBD2 (1X9, 11X12, 3X11) foot pull-out alarm | XWB | 2023-7-28 |
| 57 | VJT.04.57 | 1. Proofread the content and organize the format to standardize it. | PZQ | 2023-10-23 |
| 58 | VJT.04.58 | 1. Added B gear indication to the vehicle status at 0200 | PZQ | 2023-10-24 |
| 59 | VJT.04.59 | 1. Add new energy data items: vehicle status, insulation resistance, battery health status, maximum single cell voltage, minimum single cell voltage, unit voltage difference, power level | JJ | 2023-10-27 |
| 60 | VJT.04.60 | 1. Add a new function ID: 0x0210, which is used to encapsulate and report BMS data stream. | PZQ | 2023-12-02 |
| 61 | VJT.04.61 | 1. Function ID: 0x0210, adjust the content. | PZQ | 2023-12-04 |
| 62 | VJT.04.62 | 1. Function ID: 0x0210, single cell voltage unit adjustment, single cell temperature description adjustment. | PZQ | 2023-12-04 |
| 63 | VJT.04.63 | 1. Add 510F reference torque value to truck data stream | Water | 2024-04-22 |
| 64 | VJT.04.64 | - Added description of hybrid vehicle data items in the extended data items for sedans: 0x6907 Cruising range. If it is a hybrid car, it indicates the cruising range of pure electric + fuel. If it is a fuel car, it indicates the cruising range of the fuel car. 0x6908 Remaining power, indicating the remaining power of the hybrid vehicle; 0x6909 Charging pile status and 0x690A charging status both indicate the status of hybrid vehicles; 2. In the 0x8105 terminal control, the vehicle control command is added: 0xa8 unlock and power on command; 0xa9 locks and cuts off oil and power. 3. In the 0x0200 position information report, a new gear position "L" is added to the gear status in the basic data stream appendix 0x0011 vehicle status. | PZQ | 2024-07-26 |
| 65 | VJT.04.65 | Added 0200 extended data item, function id: 0x300E, 3.80 high-speed rail tire pressure data table | HYD | 2024-08-15 |
| 66 | VJT.04.66 | - In the 0200 data stream, add the YF customized 5 high and 1 low i/O port detection function description, from ID (0x0022), adjust the function ID to (0x0026). - Adjust the record format | PZQ | 2024-08-29 |
| 67 | VJT.04.67 | WIFI data flow table adjustment | HYD | 2024-09-02 |
| 68 | VJT.04.68 | Added customized agreements related to Thailand projects 1. In the 0200 extended data item, modify the function ID: 0x300D to distinguish between Thailand project temperature and other project temperature , and add function ID: 0x3013, 0x3014, 0x3015; 2. Add 0252 card swiping information data; | HYD | 2024-09-10 |
| 69 | VJT.04.69 | Modified the attached car extended data stream by adding data items 0x6F05, 0x6F06, 0x6F07, 0x6F08 | HYD | 2024-09-13 |
| 70 | VJT.04.70 | - 1. Add 0x41 in 0900 and 8900 for platform downstream transparent transmission and terminal upstream transparent transmission - 2. Added second ADC acquisition "0x3016 Analog Signal Input 2" in the extended data item of 0200 extended data stream - 3. Add a status flag in the 0200 data stream to indicate differential or non differential positioning flags | PZJ | 2024-09-14 |
| 71 | VJT.04.71 | - 1. Added 0x42 in 0900 and 8900 to distinguish between downstream and upstream transparent transmission of two serial ports - 2. Added 0x202C and 0x202D in 8103 for configuring serial port baud rate | HYD | 2024-09-24 |
| 72 | VJT.04.72 | According to the 808-2013 standard protocol, supplement the 18-21 bits in the 0200 status flag definition | HYD | 2024-10-12 |
| 73 | VJT.04.73 | 1. Added key power switch command in 0x8105 interrupt control command, 0xAB | PZJ | 2024-10-12 |
| 74 | VJT.04.74 | 0200 basic data stream adds GPS positioning solution status 0x0027 | HYD | 2024-10-16 |
| 75 | VJT.04.75 | 0200 truck expansion data stream adds dedicated data items for the auxiliary engine of sanitation vehicles 0x5210~0x5216 | HYD | 2024-10-18 |
| 76 | VJT.04.76 | 1. New low oil level alarm command 0x0410 2. Update on the description of the load of the auxiliary engine and the torque percentage threshold of the environmental sanitation vehicle engine | CXQ | 2024-10-22 |
| 77 | VJT.04.77 | 1. In the 0200 vehicle status, the newly added key power supply status is used to indicate the disconnection and continuity of the key power supply | PZJ | 2024-11-09 |
| 78 | VJT.04.78 | 1. Remove 0405 and 0406 (4-5 pin plug-in alarm) from the 0200 alarm ID | HYD | 2024-11-11 |
| 79 | VJT.04.79 | 1. 0x8105 Added BMS lock control command 2. 0x0200 adds BMS status, including BMS working status and BMS lock status 3. 0x0200 Modify the accuracy of the highest cell voltage, lowest cell voltage, and cell voltage difference inside to 0.001V 4. The number of individual battery packs in the 0x0210 expansion has been increased from 128 to 384 | PZJ | 2024-11-14 |
| 80 | VJT.04.80 | - 1. Newly added 0705 command for reporting collected CAN data streams - (According to the 808-2013 Ministry Standard Agreement) 2. Add a sub function ID of 0x2050, supporting configuration and querying, CAN ID configuration filter table - 3. Add a sub function ID 0x202E that supports configuration and querying. When ACC ON(via ACC line), the CAN transparent transmission reporting interval is increased - 4. Add a sub function ID 0x202F that supports configuration and querying. When ACC OFF (via ACC line), the CAN transparent transmission reporting interval is increased - 5. Add a sub function ID 0x2030, supporting configuration and query, buzzer control (can only be controlled when ACC is ON) | CXQ | 2024-12-1 |
| 81 | VJT.04.81 | - 1. In the 0200 basic data stream, add 0x0028, the running time of the device startup, which accumulates from 0 after the device is powered off and restarted | PZJ | 2025-01-08 |
| 82 | VJT.04.82 | - 1. Add key detection to the vehicle status table in the 0200 basic data stream to check for the presence of a status | PZJ | 2025-01-18 |
| 83 | VJT.04.83 | - 1. Add ternary catalytic data item to 0200 truck data stream | JJH&PZJ | 2025-01-20 |
| 84 | VJT.04.84 | - 1.Add Driver swipe card data pack packet to 0200 extended peripheral data stream | CXQ | 2025-5-29 |
| 85 | VJT.04.85 | - 1. Add average voltage, average temperature value, accelerator pedal position, and battery pack capacity to the 0200 Appendix_New Energy Vehicle Data Flow | CXQ | 2025-07-29 |
| 88 | VJT.04.88 | - Add power consumption algorithm, add remaining power (in KWH) | CXQ | 2025-12-11 |

Order  record

-V JT . 04 .085 -A	1

1. Introduction	12

1.1 Purpose	12

1.2 Terms and Definitions	12

1.3 Abbreviations	12

1.4 Protocol Basics	13

1.5 Message composition	14

1.6 Communication Connection	16

1.7 Message Processing	16

1.8 SMS message processing	17

1.9 Protocol Classification	17

2. Data Format	22

2.1 [0001] Terminal General Response	22

2.2 [ 8001] Platform General Response	22

2.3 [ 0002 ] Terminal heartbeat	22

2.4 [ 0100 ] Terminal registration	23

2.5 [ 0003 ] Terminal Logout	23

2.6 [ 0102 ] Terminal Authentication	23

2.7 [ 0200 ] Location information reporting	24

2.8 [ 0704 ] Batch reporting of location information	24

2.9 [ 020 A ] CAN broadcast data stream report	24

2.10 [ 02 10 ] New Energy Vehicle BMS Data Flow Report	25

2.11 [ 0900 ] Data uplink transparent transmission	25

Vehicle trip data package              0xF1	25

Vehicle fault code data package            0xF2	25

Vehicle sleep entry data packet          0xF3	25

Vehicle sleep wake-up data packet          0xF4	25

MCU upgrade status feedback package         0xF6	25

Suspected collision alarm description package          0xF7	25

2.12 [0205] Actively report version information ( non-ministry standard )	26

2.13 [ 8103 ] Set terminal parameters	26

2.14 [ 8104 ] Query terminal parameters	26

2.15 [ 8201 ] Location information query	27

2.16 [ 8300 ] Text message delivery	27

2.17 [ 6006] Text message reply	27

2.18 [ 8105 ] Terminal Control	28

2.19 [ 0108 ] Terminal upgrade result notification	28

2.20 [ 0702 ] Driver identity information collection	28

2.21 [ 8202 ] Temporary position tracking control	29

2.22  [ 0252 ] Terminal report driving license data	30

3. Appendix 1:	31

3.1 Appendix_Terminal General Response	31

3.2 Appendix_Platform General Response	31

3.3 Appendix_Terminal registration message body	31

3.4 Appendix_Terminal registration response message body	31

3.5 Appendix_Terminal authentication message body	32

3.6 Appendix_Terminal parameter message body	32

3.7 Appendix_Parameter Item Format	32

3.8 Appendix_Terminal parameter setting definition	32

3.9 Appendix WIFI parameter list	38

3.10 Appendix_Rapid acceleration parameters	38

3.11 Appendix_Rapid deceleration parameters	38

3.12 Appendix_Sharp Turn Parameters	38

3.13 Appendix_Terminal upgrade data package	39

3.14 Appendix_Platform upgrade data packet response	39

3.15 Appendix_Trailer alarm parameters	39

3.16 Appendix_Collision Alarm Parameter Package	39

3.17 Appendix_Privilege Number List	40

3.18 Appendix_Query terminal parameter response message body	40

3.19 Appendix_Terminal control message body	40

3.20 Appendix_Terminal Control Command Word Description Table	40

3.21 Appendix_Terminal control response message body	41

3.22 Appendix_Terminal Control Response Appendix	41

3.23 Appendix_Terminal Control Response Result Appendix​	42

3.24 Appendix_Command parameter format	43

3.25 Appendix_Driver Information Collection Appendix	43

3.26 Appendix_Temporary location tracking control message body	44

3.27 Appendix_Terminal upgrade result data package	45

3.28 Appendix_Location Information Query Response Message Body Data Format	46

3.29 Appendix_Location Data Batch Reporting Package	46

3.30 Appendix_Position Batch Reporting Data Item Format	46

3.31 Appendix_Location data information body	46

3.32 Appendix_Status Flag Definition	47

3.33 Appendix_Alarm flag definition	47

3.34 Appendix_Additional location information table	49

3.35 Appendix_Additional Information Definition	49

3.36 Appendix_Basic data flow​	51

3.37 Appendix_Car extended data flow	54

3.38 Appendix_Truck Extended Data Flow	56

3.39 Appendix_New Energy Vehicle Data Flow	60

3.40 Schedule Extending peripheral data flow	62

3.41 Appendix_Alarm command ID and description items	63

3.42 Appendix_Base station data flow	65

3.43 Appendix_Basic data items: Accelerometer	66

3.44 Appendix_Basic data items: Total mileage format table	66

3.45 Appendix_Basic data items: Accumulated mileage 2 format table	67

3.46 Appendix_Basic data items: Total fuel consumption format table	67

3.47 Appendix_Basic data items: Accelerometer	68

3.48 Appendix_Basic data items: Protocol type table	68

3.49 Appendix_Basic data items: Vehicle status table	68

3.50 Appendix_Alarm Description :  Idle alarm description	70

3.51 Appendix_Alarm Description :  Overspeed alarm description	70

3.52 Appendix_Alarm Description :  Fatigue driving alarm description	70

3.53 Appendix_Alarm Description :  Water temperature too high alarm description	70

3.54 Appendix_Extended peripheral data : H600 video status information table	71

3.55 Appendix_Extended peripheral data : H600 input signal	73

3.56 Appendix_Extended peripheral data: Tire pressure data table	73

3.57 Appendix_Load Cell Data Sheet	74

3.58 Appendix_External oil sensor data sheet	75

3.59 Appendix_Fire truck 6-way data collection data table	75

3.60 Appendix_Version Information Package	76

3.61 Appendix_Version Information Packet Response	76

3.62 Appendix_Text message body	76

3.63 Appendix_Meaning of text information flags	76

3.64 Appendix_Text message body​	77

3.65 Appendix_Data Uplink Transparent Transmission Message Body	77

3.66 Appendix_Transparent message type definition	77

3.67 a Appendix_Driving trip data package F1	78

3.68 Appendix_Driving trip data dynamic information table	78

3.69 Appendix_Fault code data package F2	79

3.70 Appendix_Sleep entry data packet F3	81

3.71 Appendix_Sleep wake - up data packet F4	81

3.72 Appendix MCU upgrade status feedback package F6	81

3.73 Appendix_Suspected Collision Alarm Description Package F7	82

3.74 Appendix_ CAN broadcast data flow	83

3.75 Appendix_New Energy Vehicle BMS Data Information Body	83

3.76 Appendix_New Energy Vehicle BMS Data Flow	83

3.77 Appendix_New Energy Vehicle BMS Data Stream : Single Battery Pack Voltage Data Table	84

3.78 Appendix_New Energy Vehicle BMS Data Stream : Single Battery Pack Temperature Data Table	85

3.79 Appendix_WIFI data flow	86

3.80 Appendix_Extended peripheral data: High - speed rail tire pressure data table	86

3.81 Schedule_Input and Output Status Table	87

3.82 Schedule_Driving Licence Data Packets(Nbytes)	89

3.83 Track 1-3 information※	90

4. Appendix 2: Sample	90

4.1 Example of escape function:	90

4.2 An example of a reverse semantic function:	91

4.3 [ 0200 ] Location data analysis details	92

4.4 [ 0900 ] Uplink data analysis details	92

4.5 [ 8300 ] Text information data analysis details	92

mileage 2 format table 	55

3.46 Appendix_Basic data items: Total fuel consumption format table 	55

3.47 Appendix_Basic data items: Accelerometer 	56

3.48 Appendix_Basic Data Items: Protocol Type Table 	56

3.49 Appendix_Basic data items: Vehicle status table 	56

3.50 Appendix_Alarm Description :  Idle alarm description 	58

3.51 Appendix_Alarm Description :  Speeding alarm description 	58

3.52 Appendix_Alarm Description :  Fatigue driving alarm description 	58

3.53 Appendix_Alarm Description :  Water temperature too high alarm description 	58

3.54 Appendix_Extended peripheral data : H600 video status information table 	59

3.55 Appendix_Extended peripheral data : H600 input signal 	60

3.56 Appendix_Extended Peripheral Data : Tire Pressure Data Table 	60

3.57 Appendix_Load Sensor Data Sheet 	61

3.58 Appendix_External Oil Sensing Data Table 	62

3.59 Appendix_Fire truck 6-way data collection data table 	62

3.60 Appendix_Version Information Package 	63

3.61 Appendix_Version Information Packet Response 	63

3.62 Appendix_Text message body 	63

3.63 Appendix_Text Information Flag Meaning 	63

3.64 Appendix_Text message body 	63​

3.65 Appendix_Data Uplink Transparent Transmission Message Body 	64

3.66 Appendix_Transparent message type definition 	64

3.67 a Schedule_Driving Trip Data Pack F1 	65

3.68 Appendix_Driving trip data dynamic information table 	65

3.69 Appendix_DTC Data Pack F2 	66

3.70 Schedule_Sleep Entry Data Packet F3 	67

3.71 Appendix_Sleep Wake- up Data Packet F4 	67

3.72 Appendix MCU upgrade status feedback package F6 	67

3.73 Appendix_Suspected Collision Alarm Description Package F7 	68

3.74 Appendix_CAN broadcast data flow 	68

3.75 Appendix_New Energy Vehicle BMS Data Information Body 	69

3.76 Appendix_New Energy Vehicle BMS Data 	Flow 69

3.77 Appendix_New Energy Vehicle BMS Data Flow : Single Cell Battery Pack Voltage Data Table 	69

3.78 Appendix_New Energy Vehicle BMS Data Stream : Single Battery Pack Temperature Data Table 	70

3.79 Appendix_WIFI data flow 	71

3.80 Appendix_Extended Peripheral Data : High- speed Rail Tire Pressure Data Table 	71

3.81 Schedule_Input and Output Status Table 	72

3.82 Schedule_Driving License Data Packets(Nbytes) 	73

3.83 Track 1-3 information※ 	74

4. Appendix 2: Sample 	74

4.1 Escape function example: 	74

4.2 Negative semantic function example: 	75

4.3 [ 0200 ] Location data analysis details 	76

4.4 [ 0900 ] Uplink data analysis details 	76

4.5 [ 8300 ] Text information data analysis 	details 76

Introduction

Purpose

This document expands OBD related functions based on JT /T 808 departmental standard protocol.

JT /T 808 Departmental Standard Protocol: Specifies the communication protocol and data format between the on-board terminal ( hereinafter referred to as the terminal ) of the road transport vehicle satellite positioning system and the supervision / monitoring platform ( hereinafter referred to as the platform ) , including the protocol basis, communication connection, message processing, protocol classification and description, and data format.

OBD function: as an extended data function of the departmental standard protocol .

Terms and Definitions

a) Abnormal data communication link

The wireless communication link is disconnected or temporarily suspended ( such as during a call ) .

b ) Register     		The terminal sends a message to the platform to inform it that it is installed on a certain vehicle.

c ) Unregister   		The terminal sends a message to the platform to inform it to be removed from the vehicle on which it is installed.

d ) Authentication  	When the terminal is connected to the platform, it sends a message to the platform to verify its identity.

e) Location reporting strategy   Report at fixed times, fixed intervals, or a combination of both.

f) Location reporting program    Rules for determining the intervals for periodic reporting based on relevant conditions.

g) Additional points report while turning

The terminal sends a position information report message when it determines that the vehicle is turning. The sampling frequency is not less than 1Hz , and the vehicle azimuth change rate is not less than 15°/s.

Month . Last at least 3 seconds .

h) Answering strategy   Rules for the terminal to automatically or manually answer incoming calls.

i) SMS text alarm       When the terminal alarms, a text message is sent via SMS .

j) event item

The event item is set from the platform to the terminal , consisting of an event code and an event name . When the driver encounters a corresponding event , he operates the terminal to trigger an event report.

Send to the platform.

Abbreviations

APN — Access Point Name

GZIP — a GNU Free Software file compression program (GNUzip)

LCD —Liquid crystal display

RSA — an asymmetric cryptographic algorithm ( developed by Ron Rivest , Adi Shamirh , and Len Adleman , and named after them )

SMS — short message service (short message protocol)

TCP — Transmission Control Protocol

TTS — text to speech

UDP — User Datagram Protocol

VSS — vehicle speed sensor

Protocol Basics

1.4.1 Communication method

The communication method adopted by the protocol should comply with the relevant provisions of JT/T 794. The communication protocol adopts TCP or UDP , the platform acts as the server , and the terminal acts as the client. When the data communication link is abnormal , the terminal can communicate by SMS message.

1.4.2 Data Types

Data types used in protocol messages :

| Data Types | Description and requirements |
| --- | --- |
| BYTE | Unsigned single-byte integer ( byte , 8 bits ) |
| WORD | Unsigned double-byte integer ( word , 16 bits ) |
| DWORD | Unsigned four-byte integer ( double word , 32 bits ) |
| BYTE[n] | n bytes |
| BCD[n] | 8421 code , n bytes |
| STRING | GBK encoding uses a 0 terminator . If there is no data , a 0 terminator is placed. |

1.4.3 Transmission rules

The protocol uses big -endian network byte order to transfer words and double words.

The agreement is as follows :

——Byte transmission convention : transmission in the form of byte stream ;

——Word (WORD) transmission convention : first transmit the high eight bits , then transmit the low eight bits;

——Double word (DWORD) transmission convention : first transfer the high 24 bits , then transfer the high 16 bits , then transfer the high eight bits , and finally transfer the low eight bits.

Message composition

1.5.1 Message Structure

Each message consists of an identification bit, a message header, a message body, and a checksum. The message structure diagram is shown in Figure 1 :

| Identification bit | Function ID | Message Header | Message body | Check code | Identification bit |
| --- | --- | --- | --- | --- | --- |

Figure 1 Message structure

| GPRS data packet format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Identification bit | Function ID | Message Header |  |  |  |  | Message Pack | check | Identification bit |
| Identification bit | Function ID | Message attributes | Terminal phone number | Message serial number | Message Packet Encapsulation Item |  | Message Pack | check | Identification bit |
| 1 | 2 | 2 | 6 Bytes | 2 | Total number of message packets (2) | Message packet sequence number (2) | N | 1 | 1 |
| 0x7e |  | LEN | BCD[12] |  |  |  |  |  | 0x7e |

1.5.2 Identification bit

It is represented by Ox7e . If 0x7e appears in the checksum, message header, or message body , it must be escaped . The escape rules are defined as follows :

0x7e<——>0x7d followed by 0x02 ;

0x7d<——>0x7d is followed by 0x01 .

The escape process is as follows :

When sending a message : message encapsulation -> calculate and fill in the checksum -> escape ;

When receiving a message : escape and restore -> verify the check code -> parse the message.

Example :

Send a packet with the content            0x30 0x7e 0x08 0x7d 0x55            Data packets ,

The encapsulation is as follows : 0x7e .... 0x30 7d 0x02 0x08 0x7d 0x01 0x55 Xor 0x7e

1.5.3 Message Header

Message header content :

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Message body attributes | WORD | The message body attribute format structure diagram is shown in Figure 2 |
| 2 | Terminal phone number | BCD[6] | The conversion is based on the terminal's own mobile phone number after installation. If the mobile phone number is less than 12 digits , add digits in the front . For mainland mobile phone numbers, add 0 digits . For Hong Kong, Macau and Taiwan, add digits according to their area codes. |
| 8 | Message serial number | WORD | 0 in the order of sending and accumulate |
| 1 0 | Message Packet Encapsulation Item |  | If the relevant flag in the message body attribute determines the message subpackaging , then this item has content , otherwise it does not have this item |

Message body attributes :

The message body attribute format structure diagram is shown in Figure 2 :

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reserve |  | Subcontracting | Data encryption method |  |  | Message body length |  |  |  |  |  |  |  |  |  |

Figure 2 Message body attribute format structure diagram

Data encryption method :

—— bit10-bit12 is the data encryption identification bit;

——When these three bits are all 0 , it means the message body is not encrypted;

——When the 10th bit is 1 , it means that the message body is encrypted by the RSA algorithm;

——When the 12th bit is 1 , it means that the message body is encrypted by the SM4 algorithm;

The SM4 algorithm only encrypts the message body in the 0200 position report data and the 0704 batch report data, where the message body length is the encrypted length.

SM4 encryption, all server downstream data does not need to be encrypted.

——Other reservations.

Subcontracting :

13th bit in the message body attribute is 1 , it indicates that the message body is a long message and is sent in packets . The specific packet information is determined by the message packet encapsulation item.

If the 13th bit is 0 , there is no message packet encapsulation item field in the message header.

Message package encapsulation content

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Total number of message packets | WORD | The total number of packets after the message is divided |
| 2 | Package number | WORD | Start from 1 |

1.5.4 Verification Code

The check code refers to the XOR of the next byte starting from the function ID until the byte before the check code , occupying one byte.

Communication Connection

1.6.1 Connection Establishment

The daily data connection between the terminal and the platform can be TCP or UDP . After the terminal is reset, it should establish a connection with the platform as soon as possible.

connection is established , the terminal authentication message is immediately sent to the platform for authentication.

1.6.2 Maintaining the connection

After the connection is established and the terminal authentication is successful , the terminal should periodically send a terminal heartbeat message to the platform .

The terminal sends a platform general response message , and the sending period is specified by the terminal parameters.

1.6.3 Disconnection

Both the platform and the terminal can actively disconnect according to the TCP protocol , and both parties should actively determine whether the TCP connection is disconnected.

The platform determines the TCP connection is disconnected:

——According to the TCP protocol, the terminal actively disconnects;

——A terminal with the same identity establishes a new connection , indicating that the original connection has been disconnected;

——No message from the terminal , such as terminal heartbeat, is received within a certain period of time .

the terminal determines that the TCP connection is disconnected:

——According to the TCP protocol, the platform actively disconnects;

——the data communication link is disconnected;

——The data communication link is normal and no response is received after reaching the number of retransmissions.

Message Processing

1.7.1 TCP and UDP message processing

1.7.1.1 Messages sent by the platform

All messages sent by the platform require the terminal to respond . The responses are divided into general responses and special responses , which are determined by each specific functional protocol. After the sender waits for the response for a timeout , the message should be resent. The response timeout and the number of retransmissions are specified by the platform parameters . The response timeout and the number of retransmissions after each retransmission are also specified by the platform parameters . The calculation formula for the response timeout after each retransmission is shown in formula (1):

T N+1 =T N *(N+1)…………(1)

Where:

T N+1 ——Response timeout after each retransmission;

T N ——The previous response timeout;

N ——Number of retransmissions.

1.7.2 Messages sent by the terminal owner

1.7.2.1 Data communication link is normal

When the data communication link is normal , all messages sent by the terminal require the platform to respond . The responses are divided into general responses and special responses , which are determined by the specific functional protocols. After the terminal waits for the response for a timeout , the message should be resent. The response timeout and the number of retransmissions are specified by the terminal parameters . The response timeout after each retransmission is calculated according to formula (1) . For the critical alarm message sent by the terminal , if no response is received after the number of retransmissions is reached , it should be saved. In the future, the saved critical alarm message should be sent before sending other messages.

1.7.2.2 Data communication link abnormality

When the data communication link is abnormal , the terminal shall save the position information report message to be sent, and send the saved message immediately after the data communication link returns to normal.

SMS message processing

the terminal communication mode is switched to the SMS message mode of the GSM network , the PDU eight-bit encoding method is used . For messages longer than 140 bytes , they should be packetized in accordance with the GSM network's SMS service specification GSM 03.40 .

The response, retransmission and storage mechanism of SMS messages is the same as that in 6.1 , but the response timeout and retransmission times shall be processed according to the relevant setting values of parameter ID 0x0006 and 0x0007 .

Protocol Classification

Overview

The following describes the protocol by functional classification. Unless otherwise specified , TCP communication is used by default . The communication protocol between the vehicle terminal and the external device is shown 	in Appendix A. The message comparison table of message names and message IDs in the protocol is shown in Appendix B.

1.9.1 Terminal Management Protocols

1.9.1.1 Terminal Registration / Deregistration

If the terminal is not registered , it should be registered first . After successful registration, the terminal will obtain the authentication code and save it . The authentication code is used when the terminal logs in.

Before the vehicle needs to remove or replace the terminal , the terminal should perform the deregistration operation to cancel the corresponding relationship between the terminal and the vehicle.

If the terminal chooses to send terminal registration and terminal deregistration messages via SMS , the platform shall send a terminal registration response to the terminal deregistration via SMS .

Reply to the terminal logout by sending the platform's general response via SMS .

1.9.1.2 Terminal Authentication

the terminal is registered , it should be authenticated immediately after establishing a connection with the platform . The terminal shall not send other messages before the authentication is successful.

The terminal performs authentication by sending a terminal authentication message , and the platform replies with a platform general response message.

1.9.1.3 Setting / Querying Terminal Parameters

The platform sets the terminal parameters by sending a set terminal parameter message , and the terminal replies with a terminal general response message.

with a query terminal parameter response message. Terminals under different network standards should support some unique parameters of their respective networks.

1.9.1.4 Terminal Control

The platform controls the terminal by sending terminal control messages , and the terminal replies with terminal general response messages.

1.9.2 Position / Alarm Protocol

1.9.2.1 Location Information Report

The terminal periodically sends location information report messages according to parameter settings.

According to parameter control , the terminal can send a location information report message when it determines that the vehicle is turning.

1.9.2.2 Location Information Query

queries the current location information of the specified vehicle terminal by sending a location information query message , and the terminal replies with a location information query response message.

1.9.2.3 Temporary Position Tracking Control

The platform starts / stops location tracking by sending temporary location tracking control messages . Location tracking requires the terminal to stop the previous periodic report .

The terminal replies with a terminal general response message.

1.9.2.4 Terminal Alarm

When the terminal determines that the alarm condition is met , it sends a location information report message and sets the corresponding alarm flag in the location report message .

Reply to the platform's general response message to process the alarm.

Each alarm type is described in the location information report message body. The alarm flag is maintained until the alarm condition is lifted .

A position information report message should be sent immediately and the corresponding alarm sign should be cleared.

1.9.3 Information Protocol

1.9.3.1 Text message delivery

The platform sends a message by sending a text message to notify the driver in a specified way. The terminal replies with a terminal general response message.

1.9.3.2 Event Settings and Reports

sends the event list to the terminal for storage by sending event setting messages . Drivers can enter the event list interface after encountering the corresponding event.

making a selection , the terminal sends an event report message to the platform.

The event setting message requires the terminal to reply with a terminal general response message.

The event report message requires the platform to reply with a general platform response message.

1.9.3.3 Asking questions

The platform sends a message by sending questions , and sends questions with candidate answers to the terminal , which immediately displays them . After the driver selects, the terminal sends a message to the platform.

Send a question and answer message. When a question is sent , the terminal needs to reply with a terminal general answer message.

1.9.3.4 Collecting driving record data

The platform sends a driving record data collection command message to request the terminal to upload the specified data . The message requires the terminal to reply with the driving record data.

According to the uploaded message.

1.9.3.5 Information on Demand

The platform sends the information on-demand menu setting message and sends the information on-demand item list to the terminal storage . The driver can select the on-demand /

Cancel the corresponding information service . After selecting, the terminal sends a message on demand / cancel message to the platform.

After the information service is on demand , you will regularly receive information service messages from the platform , such as news, weather forecast, etc.

The information on-demand menu setting message requires the terminal to reply with a terminal general response message.

Information on demand / cancellation message requires the platform to reply with a general platform response message.

Information service message , requiring the terminal to reply with a terminal general response message.

1.9.4 Telephone Protocol

1.9.4.1 Call Back

The platform sends a call back message , requiring the terminal to call back the specified phone number and specify whether to use monitoring mode.

( The terminal does not turn on the speaker . ) To call back , the terminal needs to reply with a general answer message.

1.9.4.2 Setting up the phone book

for the terminal by sending a set phone book message , which requires the terminal to reply with a terminal general response message.

1.9.5 Vehicle Control Protocol

The platform sends a vehicle control message to request the terminal to control the vehicle according to the specified operation. After receiving the message, the terminal immediately replies with a terminal general response message. After that, the terminal controls the vehicle 	and replies with a vehicle control response message according to the result.

1.9.6 Vehicle Management Protocol

sets the area and line for the terminal by sending messages 	such as setting circular area, setting rectangular area, setting polygonal area, setting route, etc. The terminal determines whether the alarm conditions are met based on the area and line attributes . The alarms include overspeed alarm, entry and exit area / route alarm, and insufficient 	/ excessive driving time alarm . The corresponding additional location information should be included in the location information and report message.

The range of area or route ID is 1-0XFFFFFFFF . If the set ID is the same as the existing area or route ID of the same type in the terminal , the 	existing one will be updated.

delete the areas and routes saved on the terminal through messages such 	as delete circular area, delete rectangular area, delete polygonal area, delete route, etc.

To set / delete area and route messages , the terminal needs to reply with a terminal general response message.

1.9.7 Information Collection Protocol

1.9.7.1 Collecting driver identity information data

The terminal collects the driver's identity information and uploads the data to the platform for identification , and the platform responds with a success or failure message.

1.9.7.2 Collecting electronic waybill data

The terminal collects electronic waybill data and uploads it to the platform.

1.9.7.3 Downloading driving record parameters

The platform sends a driving record parameter download command message to request the terminal to upload the specified data . The terminal needs to reply to the terminal general application.

Answer the message.

1.9.8 Multimedia Protocols

1.9.8.1 Upload multimedia event information

When a terminal actively takes photos or records audio due to a specific event , it should actively upload a multimedia event message after the event occurs . The message requires the platform to reply 	with a general response message.

1.9.8.2 Multimedia Data Upload

The terminal sends a multimedia data upload message to upload multimedia data. Each complete multimedia data needs to be preceded by a location information report message body during recording 	, which is called location multimedia data. The platform determines the receiving timeout based on the total number of packets . After receiving all data packets or reaching the timeout 	, the platform sends a multimedia data upload response message to the terminal , which confirms that all data packets have been received or requires the terminal to retransmit the specified data 	packet.

1.9.8.3 Camera shoots immediately

The platform issues a shooting command to the terminal by sending a camera immediate shooting command message , which requires the terminal to reply with a terminal general response message. 	If real-time upload is specified , the terminal uploads the camera image / video after shooting , otherwise the image / video is stored.

1.9.8.4 Recording starts

The platform sends a recording start command message to the terminal , which requires the terminal to reply with a terminal general response message. If real-time upload is specified 	, the terminal uploads the audio data after recording , otherwise the audio data is stored.

1.9.8.5 Retrieval terminal stores multimedia data and extracts

obtains the storage of multimedia data in the terminal by sending a storage multimedia data retrieval message , and the message requires the terminal to reply with a storage multimedia data retrieval response message.

According to the search results , the platform can send a storage multimedia data upload message to request the terminal to upload the specified multimedia data . The message requires the terminal to reply with a terminal general response message.

1.9.9 General Data Transmission Class

Messages that are not defined in the protocol but need to be transmitted in actual use can use data uplink transparent transmission messages and data downlink transparent transmission messages to exchange uplink and downlink data . The terminal can use 	the GZIP compression algorithm to compress longer messages and upload them using data compression reporting messages.

1.9.10 Encryption Protocol

If encrypted communication is required between the platform and the terminal , the RSA public key cryptographic system can be used . The platform informs the terminal of 	its own RSA public key by sending a platform RSA public key message , and the terminal replies with a terminal RSA public key message , and vice versa.

Data Format

[0001] Terminal General Response

[ Function Description ]: Terminal general response message body data

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 01 | Message attachment | Terminal General Response Appendix | XOR | 7E |

[ 8001] Platform General Response

[ Function Description ]: Platform general response message body data

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response Appendix | XOR | 7E |

[ 0002 ] Terminal heartbeat

[Functional description]  Terminal heartbeat packet reporting

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 02 | Message attachment | none | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0100 ] Terminal registration

[Functional description]  Terminal registration message body data

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 01 00 | Message attachment | Terminal registration message body | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 81 00 | Message attachment | Terminal registration response message body | XOR | 7E |

[ 0003 ] Terminal Logout

[Functional description]  The terminal logout message body is empty

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 03 | Message attachment | none | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0102 ] Terminal Authentication

[Functional description]  Terminal authentication message body data

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 01 02 | Message attachment | Terminal authentication message body | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0200 ] Location information reporting

[Functional description]  The location information report message body consists of basic location information and a list of additional location information items.

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 00 | Message attachment | Location data message body | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0704 ] Batch reporting of location information

[Functional description]  Batch reporting of location information

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 07 04 | Message attachment | Location Data Batch Reporting Package | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0705 ] CAN broadcast data stream report

[Functional description]  Collect CAN data streams obtained from the bus

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 07 05 | Message attachment | CAN broadcast data stream (Standard 808-2013) | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 020A ] CAN broadcast data stream report

[Functional description]  Collect CAN data streams obtained from the bus

[Customized functions]

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 0A | Message attachment | CAN broadcast data stream [Customized functions] | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0210 ] New Energy Vehicle BMS Data Flow Report

[Functional description]  Collect the new energy vehicle BMS data stream obtained from the bus, and customize the function

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 10 | Message attachment | New energy vehicle BMS data information body | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0900 ] Data uplink transparent transmission

### Vehicle trip data package              0xF1

### Vehicle fault code data package            0xF2

### Vehicle sleep entry data packet          0xF3

### Vehicle sleep wake-up data packet          0xF4

### MCU upgrade status feedback package         0xF6

### Suspected collision alarm description package          0xF7

[Functional description]  Data uplink transparent transmission message body data

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 09 00 | Message attachment | Data Uplink Transparent Transmission Message Body Appendix | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 0205 ] Actively report version information ( non-ministry standard )

[Functional description]  Contains key data such as software version, software release time, module model, total mileage, total fuel consumption, VIN code, etc.

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 05 | Message attachment | Version Information Package | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 82 05 | Message attachment | Version Information Packet Response | XOR | 7E |

[ 8103 ] Set terminal parameters

[ Function Description ] : Set the terminal parameter message body data.

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 81 03 | Message attachment | Terminal parameter message body appendix | XOR | 7E |

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 01 | Message attachment | Uplink general response | XOR | 7E |

[ 8104 ] Query terminal parameters

[ Function Description ]: The query terminal parameter message body is empty.

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 81 04 | Message attachment | null | XOR | 7E |

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 01 04 | Message attachment | Query terminal parameter response message body appendix | XOR | 7E |

[ 8201 ] Location information query

[ Function description ]: The location information query message body is empty.

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 82 01 | Message attachment | null | XOR | 7E |

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 01 | Message attachment | Location Information Query Response Data Appendix | XOR | 7E |

[ 8300 ] Text message delivery

[ Function Description ]: Send text message body data. (Send SMS settings/TTS broadcast)

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 83 00 | Message attachment | Text message body attachment​ | XOR | 7E |

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 01 | Message attachment | Uplink general response | XOR | 7E |

[ 6006 ] Text message reply

[ Function Description ]: Send text message data on the terminal device

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 60 06 | Message attachment | Text message body attachment​ | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

[ 8105 ] Terminal Control

[ Function Description ]: Terminal controls the message body data format.

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 81 05 | Message attachment | Terminal Control Message Body Appendix | XOR | 7E |

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 00 01 | Message attachment | Terminal General Response | XOR | 7E |

For some terminal control types, the control result of 0x0105 needs to be supplemented. After receiving the control command, 0x0001 is sent back to the platform to indicate reception.

After the control command is received, the control result is reported to the platform through 0x0105.

[ Up ]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 01 05 | Message attachment | Terminal control response message body | XOR | 7E |

[ 0108 ] Terminal upgrade result notification

[ information  ID ] : 0x0108 .

[ Function Description ]: After the terminal upgrade is completed, the monitoring center is notified through this command .

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 01 08 | Message attachment | Upgrade result data package | XOR | 7E |

[ 0702 ] Driver identity information collection

[ information  ID ] : 0x0702 .

[ Function Description ]: After receiving the 0X8702 command, the terminal automatically replies with the 0702 driver information collection package, or automatically reports the 0702 driver information package when signing in or signing out .

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 07 02 | Message attachment | Driver Identity Information Appendix | XOR | 7E |

[ 8202 ] Temporary position tracking control

[ information  ID]: 0x8202 .

[ Function Description ]: Temporary location tracking control message body data.

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 82 02 | Message attachment | Temporary location tracking control message body appendix | XOR | 7E |

[ 0252 ] Terminal report driving license data

[Function Description] Terminalreport driving license data when first swipe card

Uplink

[Up]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 02 52 | Message attachment | driving license Data packets | XOR | 7E |

[Down]

| Logo | Function ID | Message Header | Message body | check | Logo |
| --- | --- | --- | --- | --- | --- |
| 7E | 80 01 | Message attachment | Platform General Response | XOR | 7E |

Appendix 1:

Appendix_Terminal General Response

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding platform message |
| 2 | Response ID | WORD | The ID of the corresponding platform message |
| 4 | result | BYTE | 0 : Success / confirmation; 1 : Failure; 2 : Error message; 3 : Not supported ; 4: Executing the previous operation |

Appendix_Platform General Response

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding terminal message |
| 2 | Response ID | WORD | The ID of the corresponding terminal message |
| 4 | result | BYTE | 0 : Success / confirmation; 1 : Failure; 2 : Error message; 3 : Not supported |

Appendix_Terminal registration message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Provincial ID | WORD | Indicates the province where the terminal is installed . 0 is reserved and the platform takes the default value. The province ID uses the first two digits of the six-digit administrative division code specified in GB/T 2260 . |
| 2 | City and County ID | WORD | Indicates the city and county where the terminal is installed . 0 is reserved and the platform takes the default value. The city and county ID uses the last four digits of the sixth administrative division code specified in GB/T 2260 . |
| 4 | Manufacturer ID | BYTE[5] | Five bytes , terminal manufacturer code. |
| 9 | Terminal Model | BYTE[ 20 ] | New Beidou 20 bytes . |
| 29 | Terminal ID | BYTE[7] | Seven bytes , consisting of uppercase letters and numbers . This terminal ID is defined by the manufacturer. |
| 36 | License plate color | BYTE | License plate color , in accordance with 5.4.12 of JT/T 415-2006 |
| 37 | License Plate | STRING | Motor vehicle license plate issued by the public security traffic management department |

Appendix_Terminal registration response message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding terminal registration message |
| 2 | result | BYTE | 0 : Success; 1 : The vehicle has been registered; 2 : The vehicle does not exist in the database; 3 : The terminal has been registered; 4 : The terminal does not exist in the database |
| 3 | Authentication code | STRING | This field is only available after success. |

Appendix_Terminal authentication message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Authentication code | STRING | The terminal reports the authentication code after reconnection |

Appendix_Terminal parameter message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Total number of parameters | BYTE | N parameter items |
| 1 | Parameter list |  | Parameter item format 1, Parameter item format 2 , Parameter item format 3, ...... Parameter item format N |

Appendix_Parameter Item Format

| Fields | Data Types | Description and requirements |
| --- | --- | --- |
| Parameter ID | DWORD | Parameter ID definition and description table , see terminal parameter setting definition for details |
| Parameter length | BYTE |  |
| Parameter Value |  | DWORD or STRING . If it is a multi-value parameter , multiple parameter items with the same ID are used in the message , such as the dispatch center phone number. |

Appendix_Terminal parameter setting definition

| Parameter ID | Data Types | Description and requirements |
| --- | --- | --- |
| 0x0001 | DWORD | The terminal heartbeat sending interval , in seconds (s) |
| 0x0002 | DWORD | TCP message response timeout , in seconds (s) |
| 0x0003 | DWORD | TCP message retransmission times |
| 0x0004 | DWORD | UDP message response timeout , in seconds (s) |
| 0x0005 | DWORD | UDP message retransmission times |
| 0x0006 | DWORD | SMS message response timeout , in seconds (s) |
| 0x0007 | DWORD | Number of SMS message retransmissions |
| 0x0008-0x000F |  | reserve |
| 0x0010 | STRING | Main server APN , wireless communication dial-up access point. If the network standard is CDMA , this is the PPP dial-up number. |
| 0x0011 | STRING | Primary server wireless communication dial-up user name |
| 0x0012 | STRING | Main server wireless communication dial-up password |
| 0x0013 | STRING | Main server address , IP or domain name |
| 0x0014 | STRING | Backup server APN , wireless communication dial-up access point |
| 0x0015 | STRING | Backup server wireless communication dial-up user name |
| 0x0016 | STRING | Backup server wireless communication dial-up password |
| 0x0017 | STRING | Backup server address , IP or domain name |
| 0x0018 | DWORD | Server TCP Port |
| 0x0019 | DWORD | Server UDP Port |
| 0x001A-0x001F |  | reserve |
| 0x0020 | DWORD | Position reporting strategy , 0: regular reporting; 1: fixed-distance reporting; 2: regular and fixed-distance reporting |
| 0x0021 | DWORD | Location reporting scheme , 0: Based on ACC status; 1: Based on login status and ACC status , first determine the login status , if logged in then based on ACC status |
| 0x0022 | DWORD | The driver's non-login reporting interval , in seconds (s), > 0 |
| 0x0023-0x0026 | DWORD | reserve |
| 0x0027 | DWORD | Report interval during sleep , in seconds (s), > 0 |
| 0x0028 | DWORD | Reporting interval for emergency alarm , in seconds (s), > 0 |
| 0x0029 | DWORD | Default time reporting interval , in seconds (s), > 0 |
| 0x002A-0x002B | DWORD | reserve |
| 0x002C | DWORD | Default distance reporting interval , in meters (m), > 0 |
| 0x002D | DWORD | The driver did not log in to report the distance interval , in meters (m), > 0 |
| 0x002E | DWORD | Report distance interval in sleep mode , in meters (m), > 0 |
| 0x002F | DWORD | Report distance interval in emergency alarm , in meters (m), > 0 |
| 0x0030 | DWORD | Inflection point supplement angle , <180° |
| 0x0031-0x003F |  | reserve |
| 0x0040 | STRING | Monitoring platform phone number |
| 0x0041 | STRING | Reset phone number , you can use this phone number to call the terminal to reset the terminal |
| 0x0042 | STRING | Restore factory settings phone number , you can use this phone number to call the terminal to restore the terminal to factory settings |
| 0x0043 | STRING | Monitoring platform SMS phone number |
| 0x0044 | STRING | Receiving terminal SMS text alarm number |
| 0x0045 | DWORD | Terminal call answering strategy , 0: automatic answering; 1: automatic answering when ACC is ON , manual answering when ACC is OFF |
| 0x0046 | DWORD | The maximum call time for each call , in seconds (s), 0 means no call allowed , 0xFFFFFFFF means no limit |
| 0x0047 | DWORD | The longest call time of the month , in seconds (s), 0 means no calls allowed , 0xFFFFFFFF means no limit |
| 0x0048 | STRING | Monitoring phone numbers |
| 0x0049 | STRING | Privileged SMS numbers for regulatory platforms |
| 0x004A-0x004F |  | reserve |
| 0x0050 | DWORD | Alarm shield word. Corresponding to the alarm flag in the position information report message , if the corresponding bit is 1, the corresponding alarm is shielded. |
| 0x0051 | DWORD | Alarm text SMS switch , corresponding to the alarm flag in the location information report message , the corresponding bit is 1, then the corresponding alarm will send a text SMS |
| 0x0052 | DWORD | Alarm shooting switch , corresponding to the alarm flag in the location information report message , the corresponding bit is 1 when the camera shoots the corresponding alarm |
| 0x0053 | DWORD | The alarm shooting storage flag corresponds to the alarm flag in the location information report message . If the corresponding bit is 1, the photo of the corresponding alarm card is stored , otherwise it is transmitted in real time. |
| 0x0054 | DWORD | Critical flag , corresponding to the alarm flag in the position information report message , if the corresponding bit is 1, the corresponding alarm is a critical alarm |
| 0x0055 | DWORD | Maximum speed , in kilometers per hour (km/h) |
| 0x0056 | DWORD | Overspeed duration , in seconds (s) |
| 0x0057 | DWORD | Continuous driving time threshold , in seconds (s) |
| 0x0058 | DWORD | The cumulative driving time threshold for the day , in seconds (s) |
| 0x0059 | DWORD | Minimum rest time , in seconds (s) |
| 0x005A | DWORD | Maximum parking time , in seconds (s) |
| 0x005B-0x006F |  | reserve |
| 0x0070 | DWORD | Image/video quality , 1-10, 1 is best |
| 0x0071 | DWORD | Brightness , 0-255 |
| 0x0072 | DWORD | Contrast , 0-127 |
| 0x0073 | DWORD | Saturation , 0-127 |
| 0x0074 | DWORD | Chroma , 0-255 |
| 0x0075-0x007F | DWORD |  |
| 0x0080 | DWORD | Vehicle odometer reading , 1/10km |
| 0x0081 | WORD | The province ID of the vehicle |
| 0x0082 | WORD | The city ID where the vehicle is located |
| 0x0083 | STRING | Motor vehicle license plate issued by the public security traffic management department |
| 0x0084 | BYTE | License plate color , in accordance with 5.4.12 of JT/T415-2006 |
| 0x0090​ | BYTE | Positioning mode; 0x01: GPS, 0x02: BD, 0x03 dual mode |
| The following ID is for manufacturers only |  |  |
| 0x2001 | BYTE | Clear fault codes 0x01: Clear 0x00: Do not clear |
| 0x2002 | BYTE | Clear vehicle data 0x01: Clear 0x00: Do not clear |
| 0x2003 | BYTE | Clear driving trip data 0x01: Clear 0x00: Do not clear |
| 0x2004 | DWORD | Total fuel consumption ml |
| 0x2006 | DWORD | Water temperature alarm parameter , unit ℃ |
| 0x2007 | BYTE | Sudden acceleration parameters table |
| 0x2008 | BYTE | Sudden deceleration parameter table |
| 0x2009 | BYTE | Sharp Turn Parameters Appendix |
| 0x200A | WORD | Vehicle type , see the manufacturer's model table for details |
| 0x200B | DWORD | Low voltage alarm parameter, unit 0.1V |
| 0x200C | DWORD | Idle time too long alarm, unit S |
| 0x200D | DWORD | Alarm when positioning time is too long, unit S |
| 0x200E | STRING | Trailer Alarm Parameters Appendix |
| 0x200F | BYTE | Collision Alarm Parameters Appendix |
| 0x2010 | STRING | Privilege Number Schedule |
| 0x2011 | DWORD | Ignition threshold voltage, unit 0.1V |
| 0x2012 | WORD | Mileage type (high byte), fuel consumption type (low byte) Mileage Type: 0x00: Cancel mandatory settings 0x01: GPS 0x02: J19391 0x03: J19392 0x04: J19393 0x05: J19394 0x06: J19395 0x07: OBD instrument panel 0x08: OBD/Private Protocol 0x09: J1939A 0x0A: J1939B 0x0B: J1939C 0x0C: J1939D 0x0D: Pulse speed … 0xff: Do not change the mandatory type Fuel consumption type: 0x00: Cancel mandatory settings 0x01: J19391 0x02: J19392 0x03: J19393 0x04: J19394 0x05: J19395 0x06: OBD1 0x07: OBD2 … 0xff: Do not change the mandatory type |
| 0x2013 | WORD | Mileage factor: setting value/1000. For example: 1020 -> 1.02 |
| 0x2014 | WORD | Fuel consumption coefficient: setting value/1000. For example: 1020 -> 1.02 |
| 0x2015 | WORD | Oil density: Diesel 0# 835 Diesel 10# 840 Diesel 20# 830 Diesel 35# 820 Diesel 50# 816 Gasoline 90# 722 Gasoline 92# 725 Gasoline 95# 737 Gasoline 98# 753 |
| 0x2016 | WORD | Idle fuel consumption coefficient: setting value/1000. For example: 1020 -> 1.02 |
| 0x2017 | BYTE | 0x01: Open OBD 0x00: Disable OBD |
| 0x2018 | BYTE | The device uses the first-in-first-out method by default to send location data. 0x00: First in, first out (default) 0x01: Real-time priority |
| 0x2019 | BYTE | The triple emergency alarm requires additional data packets a few seconds before and after: the added data is mainly for 0200. 0x00-0x0A, the maximum is 10 seconds, the default is 0 seconds, which means turning off this function. |
| 0x201A | BYTE | Read fault code instructions: 0x01: Read OBD fault code and report it via 0900 or F2. 0x00: Do not read fault codes. |
| 0x201B | STRING | WIFI parameter table |
| 0x201C | DWORD | The unit is seconds. The minimum duration of sleep wake-up is 5 minutes, which is 300 seconds. |
| 0x201D | WORD | Rapid acceleration threshold, unit: mg |
| 0x201E | WORD | Rapid deceleration threshold, unit: mg |
| 0x201F | WORD | Sharp turn speed threshold, unit: mg |
| 0x2020 | BYTE[2] | Emergency brake parameters: Specific principle description is as per the alarm reporting section BYTE[0]: Speed difference threshold default 9 km/h BYTE[1]: vehicle speed is greater than a certain speed, default 0 km/h |
| 0x2021 | BYTE | Emergency brake parameters: Specific principle description is as per the alarm reporting section Speed difference threshold default 18 km/h |
| 0x2022 | WORD | Overspeed parameters: Specific principle description is as per the alarm reporting section Engine speed threshold 2400 rpm |
| 0x2023 | WORD | PTO idle speed parameters: Specific principle description is as per the alarm reporting section Engine speed threshold 1000 rpm |
| 0x2024 | BYTE | Log upload, 1 is on (automatically shut down after 20 minutes of uploading) 0 is off |
| 0x2025 | WORD | Flame-out delay time, unit S |
| 0x2026 | BYTE | Set whether the ACC line is valid; invalid: 0x00; valid: 0x01 |
| 0x2027 | WORD | OBD data stream sending interval, which means the interval between frames is no less than 70 milliseconds. The default value is 70 milliseconds |
| 0x2028 | WORD | OBD data transmission interval, 0200 data, default 60S |
| 0x2029 | BYTE [n] | Bluetooth authorization code: a string of hexadecimal numbers, up to 50 bytes. W/R |
| 0x202A | STRING | Bluetooth name: greater than 8 bytes, maximum 35 bytes. W/R |
| 0x202B | STRING | Bluetooth MAC: such as 44A6E5148CFE. |
| 0x202C | BYTE | Serial port 1 baud rate configuration: 1：‌9600 2：‌19200 3：‌38400 4：‌57600 5：‌115200 Other values: 115200 |
| 0x202D | BYTE | Serial port 2 baud rate configuration: 1：‌9600 2：‌19200 3：‌38400 4：‌57600 5：‌115200 Other values: 115200 |
| 0x202E | WORD | When ACC ON (via ACC line), CAN transparent transmission reporting interval is 0x0705, default 5s |
| 0x202F | WORD | When ACC OFF (via ACC line), CAN transparent transmission reporting interval is 0x0705, default 60s |
| 0x2030 | BYTE | 0x01: Open buzzer 0x00: close buzzer ( The control is only effective when the ACC state is on ) |
| 0x2050 | BYTE[N] | CAN ID configuration filter table |

Appendix_CAN ID configuration filter table

| Serial number | content | byte count | data type | description |
| --- | --- | --- | --- | --- |
| 0 | Total number of filtering groups | 1 | Byte | Total number of filtering groups, supporting up to 14 CAN filtering groups If it is 0xAA, it means all are reported If it is 0x00, it means that none of them will be reported |
| 1+(N-1)*8 | Filter Group 1 | 8 | Byte[8] | Rule 1, specifically explained as follows; |
| ... | ... | ... | ... |  |
| 1+(N-1)*8 | Filter Group N | 8 | Byte[8] | Rule N, specifically explained as follows; |

Rule:

8 bytes in total, CAN ID [4BYTE]+mask [4BYTE]

Example description:

0x11223344  CAN ID  [binary system]  00010001 00100010 00110011 01000100

0xFF00FF00  mask    [binary system]  11111111 00000000 11111111 00000000

The CAN ID with the first byte being 11 and the third byte being 33, regardless of the other bytes, must be packaged and reported to the server for all CAN IDs that meet this condition.

Appendix WIFI parameter list

| Serial number | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Enable | 1 | ASCII | Enable : 0 : Disable 1 : Enable |
| 1 | , | 1 | ASCII | 0x2C |
| 2 | SSID | Variable length | ASCII | WiFi SSID |
| 3 | , | 1 | ASCII | 0x2C |
| 4 | Password | Variable length | ASCII | W ifi password |

Appendix_Rapid acceleration parameters

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Rapid acceleration level | 1 | BYTE | 0X03 : high sensitivity; 0X02: medium sensitivity; 0X01: low sensitivity, 0X00: off |

Appendix_Rapid deceleration parameters

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Rapid deceleration level | 1 | BYTE | 0X03 : high sensitivity; 0X02: medium sensitivity; 0X01: low sensitivity, 0X00: off |

Appendix_Sharp Turn Parameters

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Sharp turn level | 1 | BYTE | 0X03 : high sensitivity; 0X02: medium sensitivity; 0X01: low sensitivity, 0X00: off |

Appendix_Terminal upgrade data package

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Firmware version number length | 1 | BYTE | Length of the version number string of the upgrade file |
| 1 | Firmware version number | N | BYTE [N] | String |
| 1+n | Offset address | 4 | BYTE | Identifies the data offset address of the currently requested upgrade file , starting from 0 |
| 1+n+4 | Length of data requested | 4 | BYTE | Identifies the length of the data currently being requested |

Appendix_Platform upgrade data packet response

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Firmware version number length | 1 | BYTE | Length of the version number string of the upgrade file |
| 1 | Firmware version number | N | BYTE [N] | String |
| 1+n | Total size of upgrade files | 4 | Dword |  |
| 1+n+4 | Upgrade file verification | 4 | Dword |  |
| 1+n+8 | Offset address | 4 | Dword | Identifies the data offset address of the currently requested upgrade file , starting from 0 |
| 1+n+12 | Upgrade data pack contents | N | BYTE [N] | data content currently sent |

Appendix_Trailer alarm parameters

| Serial number | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Enable | 1 | ASCII | Alarm shield: 0 no alarm 1 alarm |
| 1 | , | 1 | ASCII | 0x2C |
| 2 | Tow Spd | Variable length | ASCII | Trailer threshold speed ( unit KM/H , greater than 15 KM/H ) |
| 3 | , | 1 | ASCII | 0x2C |
| 4 | Tow Interval | Variable length | ASCII | Trailer condition satisfied duration ( Unit: seconds , greater than 2 0 seconds ) |

Appendix_Collision Alarm Parameter Package

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Collision Level | 1 | BYTE | 0X0 3: High sensitivity; 0X02: medium sensitivity; 0X01: Low sensitivity, 0x00: Off |

Appendix_Privilege Number List

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Privilege Number | 11 | ASCII | 13866668888, indicating that the number allows configuration of query parameters. |

Appendix_Query terminal parameter response message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding terminal parameter query message |
| 2 | Total number of parameters | BYTE |  |
| 3 | Parameter list |  | Parameter item format table |

Appendix_Terminal control message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Command word | BYTE | Terminal control command word description table |
| 1 | Command Parameters | STRING | The command parameter format is described in the following sections . Each field is separated by a half-width " ; ". Each STRING field is first processed according to GBK encoding before forming a message. |

Appendix_Terminal Control Command Word Description Table

| Command word | Command Parameters | Description and requirements |
| --- | --- | --- |
| 0x0 1 | Command parameter format | Wireless upgrade. Parameters are separated by semicolons. The command is as follows: " URL address ; dial-up point name; dial-up user name ; dial-up password; IP address; TCP port; UDP port; manufacturer ID ; hardware version; firmware version; time limit for connecting to the specified server" . If a parameter has no value , leave it blank. |
| 0x0 4 | none | Terminal reset |
| 0x0 5 | none | Restoring the terminal to factory settings |
| 0x90 | 1 byte+10 bytes | Oil and power off 1 byte 0x01: Oil off 10 bytes reserved |
| 0x91 | 1 byte+10 bytes | Oil and power on 1 byte 0x01: Oil on 10 bytes reserved |
| 0x92 | 1 byte+10 bytes | Ignition 1 byte 0x01: Ignition 10 bytes reserved |
| 0x93 | 1 byte+10 bytes | Flame off 1 byte 0x01: Flame off 10 bytes reserved |
| 0xA0 | 1 byte+10 bytes | Order/return the car 1 byte 0x01: order; 0x00: return the car 10 bytes reserved |
| 0xA1 | 1 byte+10 bytes | Car search 1 byte 0x01: horn; 0x02: light; 0x03: horn + light; 10 bytes reserved |
| 0xA2 | 1 byte+10 bytes | Central locking 1 byte 0x01: unlock; 0x00: lock 10 bytes reserved |
| 0xA3 | 1 byte+10 bytes | Window 1 byte 0x01: open window; 0x00: close window 10 bytes reserved |
| 0xA4 | 1 byte+10 bytes | Trunk lock 1 byte 0x01: open the trunk; 0x00: close the trunk 10 bytes reserved |
| 0xA5 | 1 byte+10 bytes | Air conditioning 1 byte 0x01: turn on the air conditioning; 0x00: turn off the air conditioning 10 bytes reserved |
| 0xA6 | 1 byte+10 bytes | Wiper 1 byte 0x01: open the wiper; 0x00: close the wiper 10 bytes reserved |
| 0xA7 | 1 byte+10 bytes | Skylight 1 byte 0x01: open skylight; 0x00: close skylight 10 bytes reserved |
| 0xA8 | 1 byte+10 bytes | Unlock and power on 1 byte 0x01: unlock and power on 10 bytes reserved |
| 0xA9 | 1 byte+10 bytes | Lock and cut off oil and power 1 byte 0x01: Lock and cut off oil and power 10 bytes reserved |
| 0xAA | 2 bytes + 9 bytes | OUT Control 1 byte 0x01: pull OUT1 high 0x00: pull OUT1 low 2 bytes 0x01: pull OUT2 high 0x00: pull OUT2 low 9 bytes reserved |
| 0xAB | 1byte+10byte | Key power switch 1-byte 0x01: No power supply; 0x00: Power supply reserved for 10 bytes explain: When the key power switch is set to no power supply, the vehicle can be successfully unlocked and locked through the central locking system, but the presence of the key cannot be detected, which means the ignition cannot be started; When the key power switch is set to power supply, the vehicle can be successfully unlocked and locked through the central locking system, and the presence of the key can be detected, indicating ignition and start; This command is only used when it is necessary to forcibly turn off the key power supply. Under normal circumstances, the central locking unlock key is powered on, while the central locking lock lock key is not powered on. |
| 0xAC | 1byte+10byte | BMS lock control,1-byte 0x01: locked; 0x00: Unlock 10 byte reservation |
| 0xF1 | none | GSM module OTA upgrade started |

Appendix_Terminal control response message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding platform message |
| 1 | Command parameters | BYTE[N] | Terminal Control Response Appendix |

Appendix_Terminal Control Response Appendix

Only when the control command word is in the table below, you need to respond to the 0x0105 command

| Command word | Command parameters | Description and requirements |
| --- | --- | --- |
| 0x90 | 1 byte+10 bytes | Fuel and power off 1 byte terminal control response result 10 bytes reserved |
| 0x91 | 1 byte+10 bytes | 1 byte for oil and power on, 10 bytes for terminal control response result reserved |
| 0x92 | 1 byte+10 bytes | Ignition 1 byte Terminal control response result 10 bytes reserved |
| 0x93 | 1 byte+10 bytes | Flame-out 1 byte Terminal control response result 10 bytes reserved |
| 0xA0 | 1 byte+10 bytes | 1 byte for ordering/returning a vehicle, 10 bytes for terminal control response result |
| 0xA1 | 1 byte+10 bytes | 1 byte for car search, 10 bytes for terminal control response result |
| 0xA2 | 1 byte+10 bytes | Central locking 1 byte terminal control response result 10 bytes reserved |
| 0xA3 | 1 byte+10 bytes | Window 1 byte Terminal control response result 10 bytes reserved |
| 0xA4 | 1 byte+10 bytes | Trunk lock 1 byte terminal control response result 10 bytes reserved |
| 0xA5 | 1 byte+10 bytes | Air conditioner 1 byte terminal control response result 10 bytes reserved |
| 0xA6 | 1 byte+10 bytes | Wiper 1 byte terminal control response result 10 bytes reserved |
| 0xA7 | 1 byte+10 bytes | Skylight 1 byte Terminal control response result 10 bytes reserved |

Appendix_Terminal Control Response Result Appendix​

| Control response results | Data Types | Description and requirements |
| --- | --- | --- |
| 0x00 | BYTE | Control success |
| 0x01 | BYTE | Control failed ( command not supported/function not supported ) |
| 0x02 | BYTE | Control failed because the car was not turned off. |
| 0x03 | BYTE | Control failed because the handbrake was not pulled up. |
| 0x04 | BYTE | Control failed because the vehicle speed was not zero. |
| 0x05 | BYTE | Control failed because the left front door lock was open. |
| 0x06 | BYTE | Control failed because the right front door lock was open. |
| 0x07 | BYTE | Control failed because the left rear door lock was open. |
| 0x08 | BYTE | Control failed because the right rear door lock was open. |
| 0x09 | BYTE | Control failed because the left front window was open. |
| 0x0A | BYTE | Control failed because the right front window was open. |
| 0x0B | BYTE | Control failed because the left rear window was open. |
| 0x0C | BYTE | Control failed because the right rear window was open. |
| 0x0D | BYTE | Control failed because the sunroof was not closed. |
| 0x0E | BYTE | Control failed because the left front door was opened. |
| 0x0F | BYTE | Control failed because the right front door was opened. |
| 0x10 | BYTE | Control failed because the left rear door was opened. |
| 0x11 | BYTE | Control failed because the right rear door was opened. |
| 0x12 | BYTE | Control failed because the front compartment cover was open. |
| 0x13 | BYTE | Control failed because the trunk was open. |
| 0x14 | BYTE | Control failed because the reading light was on. |
| 0x15 | BYTE | Control failed because the low beam was on. |
| 0x16 | BYTE | Control failed because the high beam was on. |
| 0x17 | BYTE | Control failed because the front fog lights were on. |
| 0x18 | BYTE | Control failed because the rear fog lights were on. |
| 0x19 | BYTE | Control failed because the hazard lights were on. |
| 0x1A | BYTE | Control failed because the width lights were on. |
| 0x1B | BYTE | Control failed because the turn signal is on |
| 0x1C | BYTE | Control failed because the wipers were on. |
| 0x1D | BYTE | Control failed because the air conditioner is on. |
| 0x1E | BYTE | Control failed because the vehicle was not returned to P gear. |
| 0x1F | BYTE | Control failed because the vehicle was not returned to N gear. |
| 0x20 | BYTE | Because the terminal executed a control action on the vehicle, the vehicle timed out and failed to control it. |
| 0x21 | BYTE | Control failed because the door is not closed (multiple doors) |
| 0x22 | BYTE | Control failed because the door locks were not closed (multiple door locks) |
| 0x23 | BYTE | Control failed because the doors and windows were not closed (multiple doors and windows) |

Appendix_Command parameter format

| Fields | Data Types | Description and requirements |
| --- | --- | --- |
| Connection Control | BYTE | 0x00 : Switch to the designated supervision platform server . After clicking on the server, the system will enter the emergency state . In this state, only the supervision platform that issues control instructions can send control instructions including SMS: 0x0 1 : Switch back to the original default monitoring platform server and restore to normal status. |
| Dial Peer Name | STRING | Generally, it is server APN , wireless communication dial-up access point , If the network standard is CDMA , the value is the PPP connection dial-up number |
| Dial-up User Name | STRING | Server wireless communication dial-up user name |
| Dial-up password | STRING | Server wireless communication dial-up password |
| address | STRING | Server address , IP or domain name |
| TCP Port | WORD | Server TCP Port |
| UDP Port | WORD | Server UDP Port Hidden function 0xAA: Switch to FTP upgrade server Hidden function 0xBB: switch to TCP upgrade server |
| Manufacturer ID | BYTE[5] | Terminal Manufacturer Code |
| Regulatory platform authentication code | STRING | The authentication code issued by the supervision platform is only used for authentication after the terminal is connected to the supervision platform . The original authentication code is used when the terminal connects back to the original monitoring platform . |
| Hardware version | STRING | The terminal's hardware version number , which is determined by the manufacturer |
| Firmware version | STRING | The terminal's firmware version number is determined by the manufacturer. |
| URL​ | STRING | Full URL​ |
| Time limit for connecting to the specified server | WORD | Unit : Minutes . A value other than 0 means that the terminal should connect back to the original address before the expiration date after receiving the upgrade or connection command . If the value is 0 , it means that the terminal will always connect to the specified server . |

Appendix_Driver Information Collection Appendix

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | state | BYTE | 0x01: Professional qualification certificate IC card is inserted (driver goes to work); |
|  |  |  | 0x02: The IC card of the professional qualification certificate is removed (the driver is off duty). |
| 1 | time | BCD[6] | Card insertion/removal time, YY-MM-DD-hh-mm-ss; |
|  |  |  | The following fields are valid and filled only when the status is 0x01. |
|  |  |  | 0x00: IC card reading success; |
|  |  |  | 0x01: Card reading failed because the card key authentication failed; |
| 7 | IC card reading result | BYTE | 0x02: Card reading failed because the card is locked; |
|  |  |  | 0x03: Card reading failed because the card was pulled out; |
|  |  |  | 0x04: Card reading failed due to data verification error. |
|  |  |  | The following fields are valid only when the IC card reading result is equal to 0x00. |
| 8 | Driver name length | BYTE | n |
| 9 | Driver's Name | STRING | Driver's Name |
| 9+n | Qualification Certificate Code | STRING | The length is 20 bits, if it is less than 0x00 is added. |
| 29+n | Issuing agency name length | BYTE | m |
| 30+n | Name of issuing agency | STRING | Name of the issuing institution of the qualification certificate |
| 30+n+m | Validity period of certificate | BCD[4] | YYYYMMDD |

Appendix_Temporary location tracking control message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Time interval | WORD | The unit is seconds (s), 0 means stop tracking. Stop tracking without subsequent fields |
| 2 | Location tracking validity period | DWORD | The unit is seconds (S) . After receiving the location tracking control message, the terminal sends a location report according to the time interval in the message before the expiration of the validity period. |

Appendix_Terminal upgrade result data package

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Upgrade Type | BYTE | 0x00: Terminal 0x12: Road transport license IC card reader 0x52: Beidou 0x2A: GSM module |
| 1 | Upgrade Results | BYTE | 0x00: success 0x01: Failed (Timeout) 0x02: Cancel 0x03: NEODOWNLOAD: NULL 0x04: NEODOWNLOAD:FAIL 0x05: NEOUPDATE:FAIL 0x06: NEOUPDATE:NUL L 0xF0: Do not upgrade if the version is the same 0xF1: Upgrade version attribute error 0xF2: Upgrade file verification error 0xF3: The upgrade file does not exist |

Appendix_Location Information Query Response Message Body Data Format

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Response serial number | WORD | The serial number of the corresponding information query message |
| 2 | Location information reporting |  | See, Location Data Message Body |

Appendix_Location Data Batch Reporting Package

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Number of data items (packets) | WORD | The number of included position data items (packets) N , > 0 |
| 2 | Location data item type | BYTE | 0: Normal batch data 1: Blind spot reporting |
| 3 | Location reporting data items |  | Location batch reporting data item format (Package 1) Location Batch Reporting Data Item Format (Package 2) Location batch reporting data item format (Package 3) .... Location batch report data item format ( Package N ) |

Note: Upload multiple packages at once

Appendix_Position Batch Reporting Data Item Format

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Location report data length | Word | Position report data length, N |
| 2 | Position report message body | BYTE[n] | Location data message body |

Appendix_Location data information body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Alarm sign | DWORD | For details, see the alarm flag definition table. |
| 4 | state | DWORD | For details, see the status flag definition table. |
| 8 | latitude | DWORD | The latitude value in degrees is multiplied by 10 to the sixth power , accurate to one millionth of a degree |
| 12 | longitude | DWORD | The latitude value in degrees is multiplied by 10 to the sixth power , accurate to one millionth of a degree |
| 16 | Elevation | WORD | Altitude , in meters (m) |
| 18 | speed | WORD | 1/10km/h |
| 20 | direction | WORD | 0-359, due north is 0 , clockwise |
| twenty two | time | BCD[6] | YY-MM-DD-hh-mm-ss (GMT+8 equipment reports using Beijing time) |
| 28 | Location extension list | nByte | See the Location Additional Information table for details. |

Appendix_Status Flag Definition

| Bit | state |
| --- | --- |
| 0 | 0: ACC off ; 1: ACC on |
| 1 | 0: Not positioned ; 1: Positioned |
| 2 | 0: North Latitude : 1: South Latitude |
| 3 | 0: East longitude ; 1: West longitude |
| 4 | 0: Operational status : 1: Out of service status |
| 5 | 0: longitude and latitude are not encrypted by the confidentiality plug-in ; l: longitude and latitude are encrypted by the confidentiality plug-in |
| 6-9 | reserve |
| 10 | 0: The vehicle oil circuit is normal : 1: The vehicle oil circuit is disconnected |
| 11 | 0: Vehicle circuit is normal : 1: Vehicle circuit is disconnected |
| 12 | 0: Door unlocked; 1 : Door locked |
| 13 | 0: Normal mode; 1: Maintenance mode |
| 14 | 0: WIFI off; 1: WIFI on |
| 15 | 0: Tire pressure 433 module is normal; 1: Tire pressure 433 module is abnormal |
| 16 | 0: Bluetooth is normal; 1: Bluetooth is abnormal |
| 17 | 0: The bucket truck is not lifted, 1: The bucket truck is lifted |
| 18 | 0: GPS satellites were not used for positioning; 1: Using GPS satellites for positioning |
| 19 | 0: No Beidou satellite was used for positioning; 1: Using Beidou satellite for positioning |
| 20 | 0: Not using GLONASS satellites for positioning; 1: Using GLONASS satellites for positioning |
| 21 | 0: Galileo satellite was not used for positioning; 1: Using Galileo satellite for positioning |
| 22 | 0: Differential positioning not used; 1: Using differential positioning |
| 23 -31 |  |

Appendix_Alarm flag definition

| Bit | definition | Processing instructions |
| --- | --- | --- |
| 0 | 1: Emergency alarm is triggered after the alarm switch is touched | Cleared after receiving a response |
| 1 | 1 : Overspeed alarm | The flag remains until the alarm condition is removed. |
| 2 | 1 : Fatigue driving | The flag remains until the alarm condition is removed. |
| 3 | 1 : Danger warning | Cleared after receiving a response |
| 4 | 1 : GNSS module failure | The flag remains until the alarm condition is removed. |
| 5 | 1 : GNSS antenna is not connected or is cut off | The flag remains until the alarm condition is removed. |
| 6 | 1 : GNSS antenna short circuit | The flag remains until the alarm condition is removed. |
| 7 | 1 : Terminal main power supply undervoltage | The flag remains until the alarm condition is removed. |
| 8 | 1 : The terminal main power is off | The flag remains until the alarm condition is removed. |
| 9 | 1 : Terminal LCD or display failure | The flag remains until the alarm condition is removed. |
| 10 | 1 : TTS module failure | The flag remains until the alarm condition is removed. |
| 11 | 1: Camera failure | The flag remains until the alarm condition is removed. |
| 12 | reserve |  |
| 13 | 1: Speeding warning | The flag remains until the alarm condition is removed. |
| 14-17 |  |  |
| 18 | 1: Accumulated driving time overtime for the day | The flag remains until the alarm condition is removed. |
| 19 | 1 : Overtime parking | The flag remains until the alarm condition is removed. |
| 20 | 1 : Enter and exit the area | Cleared after receiving a response |
| twenty one | 1: Access routes | Cleared after receiving a response |
| twenty two | 1: Insufficient / too long driving time on the road segment | Cleared after receiving a response |
| twenty three | 1: Route deviation alarm | The flag remains until the alarm condition is removed. |
| twenty four | 1 : Vehicle VSS failure | The flag remains until the alarm condition is removed. |
| 25 | 1 : Abnormal vehicle fuel level | The flag remains until the alarm condition is removed. |
| 26 | 1 : Vehicle stolen ( via vehicle immobilizer ) | The flag remains until the alarm condition is removed. |
| 27 | 1 : Illegal ignition of vehicle | Cleared after receiving a response |
| 28 | 1 : Illegal displacement of vehicle | Cleared after receiving a response |
| 29-31 | reserve |  |

Appendix_Additional location information table

| Fields | Data Types | Description and requirements |
| --- | --- | --- |
| Additional information ID | BYTE | 1-255 |
| Additional information length | BYTE | 1-255 |
| Additional Information |  | Additional Information Definition Schedule |

Appendix_Additional Information Definition

| Additional information ID (1 byte) | Additional Information Length (1 byte) | Description and requirements |
| --- | --- | --- |
| 0xE1 | 2 bytes | Speed, unit: RPM; offset: 0; range 0-8000. (Special) |
| 0xEA | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) Basic data flow appendix |
| 0xEB | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) Car Extended Data Stream |
| 0xEC | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) Truck Extension Data Flow |
| 0xED | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) New energy vehicle data items |
| 0xEE | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) Appendix of Extended Peripheral Data Items |
| 0xFA | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE) Alarm command ID and description table |
| 0xFB | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE base station data stream , reported when GPS is not positioned, customized |
| 0xFC | Nbyte | Data packet contains sub-ID (2BYTE), length (1BYTE) + data (NBYTE WIFI data stream, report when GPS is not positioned, customized |
| ... | ... | Other reservations |

Additional ID:

0XEA: The corresponding data item behind it indicates the basic data item.    Maximum length is 255;

0XEB: The corresponding data item behind it indicates the car data item,    Maximum length is 255;

0XEC: The corresponding data item behind it indicates the truck data item.    Maximum length is 255;

0XED: The corresponding data item behind it indicates the new energy vehicle data item, with a maximum length of 255;

0XEE: The corresponding data item behind it indicates the peripheral data item.    Maximum length is 255;

0XFA: The corresponding data item behind it indicates the alarm event ID.    Maximum length is 255;

0XFB: The corresponding data item behind it indicates the base station data stream.    Maximum length is 255;

0XFC: The corresponding data item behind it indicates the WiFi data stream, with a maximum length of 255;

Appendix_Basic data flow​

| Function ID field | Function ID[2] | Length[1] | Function | unit | describe |
| --- | --- | --- | --- | --- | --- |
| 0x0001-0x0FFF | 0x0001 | 4 | Reserve |  |  |
|  | 0x0002 | 4 | Reserve |  |  |
|  | 0x0003 | 5 | Total mileage data | rice | Total mileage format |
|  | 0x0004 | 5 | Total fuel consumption data | ml | Total fuel / Electricity consumption format table |
|  | 0x0005 | 4 | Total running time | Second | Total vehicle operation time |
|  | 0x0006 | 4 | Total flameout time | Second | The total duration of vehicle shutdown |
|  | 0x0007 | 4 | Total idling time | Second | Total accumulated vehicle idling time |
|  | 0x0008 | N | Mileage data table |  | Mileage reference data packet 60 bytes |
|  | 0x0009 | N | Fuel consumption data table |  | Fuel consumption reference data packet 35 bytes |
|  | 0x0010 | N | Accelerometer |  | Accelerometer |
|  | 0x0011 | 20 | Vehicle Status Table |  | Vehicle Status Table |
|  | 0x0012 | 2 | Vehicle voltage | 0.1V | 0-36V |
|  | 0x0013 | 1 | Terminal built-in battery voltage | 0.1V | 0-5V |
|  | 0x0014 | 1 | CSQ value |  | Network signal strength |
|  | 0x0015 | 2 | Model ID |  | Model ID Table |
|  | 0x0016 | 1 | OBD protocol type |  | Protocol Type Table |
|  | 0x0017 | 2 | Drive cycle tags |  |  |
|  | 0x0018 | 1 | GPS satellite reception |  | GPS positioning satellite reception |
|  | 0x0019 | 2 | GPS location accuracy | 0.01 | GPS location accuracy |
|  | 0x001A | 1 | GPS Average Signal-to-Noise Ratio | db | GPS Average Signal-to-Noise Ratio |
|  | 0x001B | 1 | GPS Antenna Status |  | 0: Antenna normal 1: Antenna open 2: Antenna short circuit (module support required) Note: Only TBOX products support this function. |
|  | 0x001D | 1 | Device unplug status (customized) |  | 0x02: Before the first positioning after the device is unplugged or powered on |
|  |  |  |  |  | Non-0x02: Other Note: To avoid the GPS track being drawn straight from the factory location to the installation location when the factory test point is shipped to the customer's first installation location |
|  | 0x001E | 4 | Mileage | rice | When the mileage type in the 0003 total mileage data is meter mileage, it is often only accurate to 1KM or 10KM, which is not conducive to mileage statistics. In order to facilitate the platform to count mileage, add an item of cumulative mileage |
|  | 0x001F | 4 | Instantaneous fuel consumption | 0.01 L/100km | Dedicated version |
|  | 0x0020 | 2 | Ignition type |  | BIT0: 1: ACC line ignition BIT1: 1: Security monitoring ignition BIT2: 1: GPS speed BIT3: 1: Voltage BIT4: 1: Engine speed BIT5: 1: ACC interrupts ignition |
|  | 0x0021 | 4 | Carbon emissions (g) | gram | Cumulative carbon emissions g, starting from the installation of the equipment |
|  | 0x0022 | 2 | Roll angular velocity (special) | 0.1dps | Bit15 indicates positive or negative, 0: positive direction; 1: negative direction Bit0-14, represents a value with a precision of 0.1 Eg; the uploaded value 0x80FF indicates a negative direction, and the angle is equal to 25.5dps, indicating 0x8000, which indicates a negative direction, 0x00FF=255, 255/10=25.5dps |
|  | 0x0023 | 2 | Pitch angular velocity (special) | 0.1dps | Bit15 indicates positive or negative, 0: positive direction; 1: negative direction Bit0-14, represents a value with a precision of 0.1 |
|  | 0x0024 | 2 | Yaw angular velocity (special) | 0.1dps | Bit15 indicates positive or negative, 0: positive direction; 1: negative direction Bit0-14, represents a value with a precision of 0.1 |
|  | 0x0025 | 5 | Accumulated mileage 2 (SWD only) | rice | Accumulated mileage 2 format table |
|  | 0x0026 | 6 | 5 high 1 low (for YF only) |  | D_IN2 (high input): Byte0 = 1: high detected; = 0: no detection D_IN3 (high input): Byte1 = 1: high detected; = 0: no detection D_IN4 (high input): Byte2 = 1: high detected; = 0: no detection D_IN5 (high input): Byte3 = 1: high detected; = 0: no detection D_IN6 (low input): Byte4 = 1: low detected; = 0: no detection D_IN7 (high input): Byte5 = 1: high detected; = 0: no detection |
|  | 0x0027 | 1 | GPS positioning solution status |  | 0: Invalid positioning 1: Ordinary positioning 2: Pseudo range differential positioning (RTD) 3: Undefined 4: RTK fixed solution 5: RTK Floating Point Solution 6: Trajectory estimation |
|  | 0x0028 | 4 | device operation time | s | The running time of device startup, accumulated from 0 after power failure and restart |

Appendix_Car extended data flow

| Function ID field | Function ID[2] | Length[1] | Function | unit | describe |
| --- | --- | --- | --- | --- | --- |
| Car data items ( Common Sections ) [0x6001-0x6FFF] | 0x60C0 | 2 | Speed | rpm | Precision: 1Offset : 0Range: 0 ~ 8000 |
|  | 0x60D0 | 1 | Speed | Km/h | Precision: 1Offset : 0Range: 0 ~ 240 |
|  | 0x62F0 | 2 | Remaining oil | % L | Remaining oil volume, unit L or % Bit15 ==0% OBD is all percentage ==1 unit L Display value is uploaded value/10 |
|  | 0x6050 | 1 | Coolant temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x60F0 | 1 | Inlet air temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x60B0 | 1 | Intake (manifold absolute) pressure | kPa | Accuracy: 1Offset : 0Range: 0 ~ 250kpa |
|  | 0x6330 | 1 | Atmospheric pressure | kPa | Accuracy: 1Offset : 0Range: 0 ~ 125kpa |
|  | 0x6460 | 1 | Ambient temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x6490 | 1 | Accelerator pedal position | % | Accuracy: 1Offset : 0Range: 0% ~ 100% |
|  | 0x60A0 | 2 | Fuel pressure | kPa | Accuracy: 1Offset : 0Range: 0 ~ 500kpa |
|  | 0x6014 | 1 | Fault code status |  | The valid range is 0~1, "0" means off and "1" means on. |
|  | 0X6010 | 1 | Number of fault codes | indivual | Precision: 1 Offset: 0 Value range: 0~255 |
|  | 0x6100 | 2 | Air flow | g/s | Precision: 0.1Offset : 0Value range: 0 ~6553.5 |
|  | 0x6110 | 2 | Absolute throttle position | % | Precision: 0.1Offset : 0Value range: 0 ~6553.5 |
|  | 0x61F0 | 2 | Time since engine start | sec | Precision: 1 Offset: 0 |
|  | 0x6210 | 4 | Fault mileage | K m | Precision: 1 Offset: 0 |
|  | 0x6040 | 1 | Calculate load value | % | Accuracy: 1Offset : 0Range: 0% ~ 100% |
|  | 0x6070 | 2 | Long term fuel trim (banks 1 and 3) | % | Precision: 0.1Offset : 0Value range: 0 ~6553.5 |
|  | 0x60E0 | 2 | First cylinder ignition timing advance angle | % | Precision: 0.1 Offset : -64 |
|  | 0x 6901 | 1 | Front brake pad wear (special) |  | 0 Normal/Otherwise display the corresponding data, unit: level |
|  | 0x 6902 | 1 | Rear brake pad wear (special) |  | 0 Normal/Otherwise display the corresponding data, unit: level |
|  | 0x6903 | 1 | Brake fluid level (special) |  | 0: Abnormal 1: Normal Other : Unable to use |
|  | 0x6904 | 2 | Engine oil level (special) | MM % | Unit: MM or % Bit15 ==0% ==1 unit MM After removing the highest BIT, the accuracy is 0.1 |
|  | 0x6905 | 4 | Left front tire pressure BYTE1 (special) Right front tire pressure BYTE2 Left rear tire pressure BYTE3 Right rear tire pressure BYTE4 | bar | 0xFF: abnormal; other values: unit: bar, precision: 0.1 0xFF: abnormal; other values: unit: bar, precision: 0.1 0xFF: abnormal; other values: unit: bar, precision: 0.1 0xFF: abnormal; other values: unit: bar, precision: 0.1 |
|  | 0x6906​​​ | 2 | Coolant level (special) |  | Precision: 1 Offset: -48 |
|  | 0x6907​​​ | 4 | Endurance (special) | km | Precision: 0.1Offset : 0 Total range of hybrid vehicle: pure electric part + fuel part Total mileage of pure fuel vehicle: fuel |
|  | 0x6908 | 1 | Remaining power (special) | % | 0% - 100% Hybrid vehicle: remaining power of pure electric part |
|  | 0x6909 | 1 | Charging status (dedicated) |  | Hybrid vehicle: pure electric partial charging status 0x0: Initial value 0x1: Not charging 0x2: AC charging 0x3: DC charging 0x4: Charging completed 0x5: Driving and Charging 0x6: Parking and charging 0x7: Invalid value |
|  | 0x 690A | 1 | Charging pile status (dedicated) |  | Hybrid vehicle: pure electric partial charging station status 0x01: Insert 0x00 : Not inserted |
|  | 0x6060 | 2 | Short-term fuel trim (cylinder banks 1 and 3) (dedicated) |  |  |
|  | 0x6340 | 4 | B1-S1 Linear or broadband oxygen sensor equivalent ratio (lambda) and current (dedicated) | N/A mA | The 4 bytes are ABCD Equivalence ratio = (A*256+B)*2/65535 Current = (C*256+D)*8/65535 |
|  | 0x6430 | 1 | Absolute load value (special) | % | Precision: 1 Offset: 0 Range: 0% ~ 100% |
|  | 0x6680 | 1 | Intake air temperature sensor (special) |  |  |
|  | 0x66f0 | 1 | Turbocharger compressor intake pressure (special) |  |  |
|  | 0x6C11​​ | 4 | Maintenance mileage (special) | km | Precision: 1 Offset: 0 |
|  | 0x6C12​​ | 1 | Accumulated collision times, total of front, rear, left and right collision times (special) | Second-rate |  |
|  | 0x6F01 | 12 | AEB1 data stream (dedicated) |  | 4-byte CAN ID plus 8-byte data stream, the terminal forwards without parsing; |
|  | 0x6F02 | 12 | AEB2 data stream (dedicated) |  | 4-byte CAN ID plus 8-byte data stream, the terminal forwards without parsing; |
|  | 0x6F03 | 12 | AEB3 data stream (dedicated) |  | 4-byte CAN ID plus 8-byte data stream, the terminal forwards without parsing; |
|  | 0x6F04 | 12 | AEB4 data stream (dedicated) |  | 4-byte CAN ID plus 8-byte data stream, the terminal forwards without parsing; |
|  | 0x6F05 | 1 | OBD Clutch Switch |  | 0X00/0X01 Shut/Open |
|  | 0x6F06 | 1 | OBD Brake Brake Switch |  | 0X00/0X01 Shut/Open |
|  | 0x6F07 | 1 | OBD Parking Brake Switch |  | 0X00/0X01 Shut/Open |
|  | 0x6F08 | 4 | Fuel used real time | ml/h |  |

Appendix_Truck Extended Data Flow

| Function ID field | Function ID[2] | Length[1] | Function | unit | describe |
| --- | --- | --- | --- | --- | --- |
| Truck Data Items 0x5001-0x6FFF | 0x60C0 | 2 | OBD speed | rpm | Precision: 1Offset : 0Range: 0 ~ 8000 |
|  | 0x60D0 | 1 | OBD speed | Km/h | Precision: 1Offset : 0Range: 0 ~ 240 |
|  | 0x62f0 | 2 | OBD remaining fuel | % L | Remaining oil volume, unit L or % Bit15 ==0% OBD is all percentage ==1 unit L Display value is uploaded value/10 |
|  | 0x6050 | 1 | OBD Coolant Temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x60F0 | 1 | OBD air intake temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x60B0 | 1 | OBD intake (manifold absolute) pressure | kPa | Precision: 1Offset : 0Range: 0 ~ 250kpa, the original protocol can only choose between 0x60B0 and 0x50B0 |
|  | 0x50B0 | 2 | OBD intake (manifold absolute) pressure | kPa | Precision: 1Offset : 0Range: 0 ~ 500kpa, for trucks, only one of 0x60B0 and 0x50B0 can be selected |
|  | 0x6330 | 1 | OBD atmospheric pressure | kPa | Accuracy: 1Offset : 0Range: 0 ~ 125kpa |
|  | 0x6460 | 1 | OBD ambient temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x6490 | 1 | OBD accelerator pedal position (Accelerator pedal) | % | Accuracy: 1Offset : 0Range: 0% ~ 100% |
|  | 0x60A0 | 2 | OBD fuel pressure | kPa | Accuracy: 1Offset : 0Range: 0 ~ 500kpa |
|  | 0x6010 | 1 | OBD fault code number | indivual | Precision: 1 Offset: 0 Value range: 0~255 |
|  | 0x5001 | 1 | OBD clutch switch |  | 0x00/0x01 Off/On |
|  | 0x5002 | 1 | OBD brake switch |  | 0x00/0x01 Off/On |
|  | 0x5003 | 1 | OBD parking brake switch |  | 0x00/0x01 Off/On |
|  | 0x5004 | 1 | OBD Throttle Position | % | Accuracy: 1Offset : 0Range: 0% ~ 100% |
|  | 0x5005 | 2 | OBD fuel usage rate (Engine fuel flow) | L/h | Accuracy: 0.05L/h Offset: 0 Value range: 0 ~ 3212.75L/h |
|  | 0x5006 | 2 | OBD Fuel Temperature | ℃ | Accuracy: 0.03125 °C Offset: -273.0°C Range: -273.0°C ~ +1734.96875°C |
|  | 0x5007 | 2 | OBD Oil Temperature | ℃ | Accuracy: 0.03125 °C Offset: -273.0°C Range: -273.0°C ~ +1734.96875°C |
|  | 0x5008 | 1 | OBD engine oil pressure | kPa | Accuracy: 4Offset : 0Range: 0 ~ 1000kpa |
|  | 0x5009 | 1 | OBD brake pedal position | % | Accuracy: 1Offset : 0Range: 0% ~ 100% |
|  | 0x500A | 2 | OBD air flow | g/s | Precision: 0.1Offset : 0Value range: 0 ~6553.5 |
|  | 0x5101 | 1 | Net engine torque | % | Accuracy: 1Offset : -125Value range: -125% ~+125% |
|  | 0x5102 | 1 | Friction torque | % | Accuracy: 1Offset : -125Value range: -125% ~+125% |
|  | 0x5103 | 2 | SCR upstream NOx sensor output value | ppm | Accuracy: 0.05Offset : -200Range : -200 ~ +3 0 12.75ppm |
|  | 0x5104 | 2 | SCR downstream NOx sensor output value | ppm | Accuracy: 0.05Offset : -200Range : -200 ~ +3 0 12.75ppm |
|  | 0x5105 | 1 | Reagent balance | % | Accuracy: 0.4Offset : 0Range: 0% ~ 100% |
|  | 0x5106 | 2 | Air intake | Kg/h | Accuracy: 0.05Offset : 0Value range: 0 ~ 3212.75 Kg/h |
|  | 0x5107 | 2 | SCR inlet temperature | ℃ | Accuracy: 0.03125 °C Offset: -273.0°C Range: -273.0°C ~ +1734.96875°C |
|  | 0x5108 | 2 | SCR outlet temperature | ℃ | Accuracy: 0.03125 °C Offset: -273.0°C Range: -273.0°C ~ +1734.96875°C |
|  | 0x5109 | 2 | DPF differential pressure | kPa | Accuracy: 0.1Offset : 0Value range: 0 ~ 6425.5 kPa |
|  | 0x510A | 1 | Engine Torque Mode |  | 0: Overspeed failure 1: Speed control 2: Torque control 3: Speed/torque control 9: Normal |
|  | 0x510B | 1 | Accelerator pedal | 1% | The displayed value is equal to the uploaded value multiplied by 0.4 |
|  | 0x510 C | 1 | Urea tank temperature | ℃ | Accuracy: 1 °C Offset: -40.0°C Range: -40.0°C ~ +210°C |
|  | 0x510 D | 4 | Actual urea injection amount | ml/h | Accuracy: 0.01Offset : 0Value range: 0 ~ 42949672.95 ml /h |
|  | 0x510 E | 4 | Cumulative urea consumption | g | Precision: 1 Offset: 0 Range: 0 ~ 4294967295 g |
|  | 0x510 F | 2 | DPF Exhaust Temperature | ℃ | Accuracy: 0.03125 °C Offset: -273.0°C Range: -273.0°C ~ +1734.96875°C |
|  | 0x5110 | 2 | Engine fuel flow | L/H | The displayed value is equal to the uploaded value multiplied by 0.05 |
|  | 0x51 11 | 1 | OBD diagnostic protocol |  | The valid range is 0~2, "0" represents IOS15765, "1" represents IOS27145, "2" represents SAEJ1939, and "0xFE" means invalid. |
|  | 0x511 2 | 1 | MIL Status |  | The valid range is 0~1, "0" means not lit, "1" means lit. "0xFE" means invalid. |
|  | 0x511 3 | 2 | Diagnostic support status |  | Each bit is defined as follows: 1 Catalyst monitoring Status Catalytic converter monitoring 2 Heated catalyst monitoring Status Heated catalyst monitoring Status 3 Evaporative system monitoring Status Evaporative system monitoring 4 Secondary air system monitoring Status Secondary air system monitoring 5 A/C system refrigerant monitoring Status A/C system refrigerant monitoring 6 Exhaust Gas Sensor monitoring Status Exhaust gas sensor monitoring 7 Exhaust Gas Sensor heater monitoring Status Exhaust gas sensor heater monitoring Status 8 EGR/VVT system monitoring EGR system and VVT monitoring 9 Cold start aid system monitoring Status Cold start aid system monitoring 10 Boost pressure control system monitoring Status Boost pressure control system 11 Diesel Particulate Filter (DPF) monitoring Status DPF monitoring 12 NOx converting catalyst and/or NOx adsorber monitoring Status Selective catalytic reduction system (SCR) or NOx adsorber 13 NMHC converting catalyst monitoring Status NMHC converting catalyst monitoring 14 Misfire monitoring support 15 Fuel system monitoring support Fuel system monitoring support 16 Comprehensive component monitoring support The meaning of each bit: 0 = not supported; 1 = supported; |
|  | 0x511 4 | 2 | Diagnostic Ready State |  | Each bit is defined as follows 1 Catalyst monitoring Status Catalytic converter monitoring 2 Heated catalyst monitoring Status Heated catalyst monitoring Status 3 Evaporative system monitoring Status Evaporative system monitoring 4 Secondary air system monitoring Status Secondary air system monitoring 5 A/C system refrigerant monitoring Status A/C system refrigerant monitoring 6 Exhaust Gas Sensor monitoring Status Exhaust gas sensor monitoring 7 Exhaust Gas Sensor heater monitoring Status Exhaust gas sensor heater monitoring Status 8 EGR/VVT system monitoring EGR system and VVT monitoring 9 Cold start aid system monitoring Status Cold start aid system monitoring 10 Boost pressure control system monitoring Status Boost pressure control system 11 Diesel Particulate Filter (DPF) monitoring Status DPF monitoring 12 NOx converting catalyst and/or NOx adsorber monitoring Status Selective catalytic reduction system (SCR) or NOx adsorber 13 NMHC converting catalyst monitoring Status NMHC converting catalyst monitoring 14 Misfire monitoring support 15 Fuel system monitoring support Fuel system monitoring support 16 Comprehensive component monitoring support Meaning of each bit: 0 = test completed or not supported; 1 = test not completed |
|  | 0x511 5 | 17 | Vehicle Identification Number (VIN) | ASCII | The vehicle identification code is a unique identifier, consisting of a 17-bit character code, which should comply with the provisions of 4.5 of GB16735. |
|  | 0x511 6 | 18 | Software calibration identification number |  | The software calibration identification number is customized by the manufacturer and consists of letters or numbers, and the character "0" is added at the end if it is insufficient. |
|  | 0x511 7 | 18 | Calibration Verification Number (CVN) |  | The calibration verification code is customized by the manufacturer and consists of letters or numbers, with the character "0" added at the end if it is insufficient. |
|  | 0x511 8 | 36 | IUPR value |  | Definitions refer to SAE J1979-DA Table G11. |
|  | 0x511A | 2 | Light absorption coefficient | 0.01m -1 |  |
|  | 0x511B | 2 | Opacity | 0.1% |  |
|  | 0x511C | 2 | Particle concentration (mass flow rate) | M g/m 3 |  |
|  | 0x511F | 1 | Real-time engine load | % | 0-100% |
|  | 0x5120 | 2 | Three way catalytic converter upstream oxygen sensor output value | V |  |
|  | 0x5121 | 2 | Three way catalyst downstream oxygen sensor output value | V |  |
|  | 0x5122 | 2 | Three way catalyst temperature sensor output value | °C |  |
|  | 0x5201 | 2 | Current powder loosening pressure (special) | 0.01Mpa |  |
|  | 0x5202 | 2 | Current left walking pressure (special) | 0.01Mpa |  |
|  | 0x5203 | 2 | Current right travel pressure (special) | 0.01Mpa |  |
|  | 0x5204 | 2 | Current powder loosening speed (special) | 1rpm |  |
|  | 0x5205 | 1 | Current fuel level alarm (dedicated) |  | 0: Normal 1: Alarm |
|  | 0x5206 | 1 | Powder loose handle left and right turn (special) |  | 0: Turn left 1: Turn right |
|  | 0x5207 | 1 | Gear status (special) |  | 0: Neutral 1: Forward 2: Reverse |
|  | 0x5208 | 1 | Locked state (dedicated) |  | 0: Walking is possible 1: Walking is locked |
|  | 0x5209 | 1 | Agricultural machinery status (special) |  | 0: Standby 1: Working |
|  | 0x520A | 2 | Total running time of the fan song engine (special) | 0.1H | Unit: Hours |
|  | 0x520B | 1 | Coolant low level alarm |  | 0: Normal 1: Alarm |
|  | 0x520C | 1 | Low engine oil warning |  | 0: Normal 1: Alarm |
|  | 0x520D | 1 | Air pressure alarm indicator |  | 0x00/0x01 Off/On |
|  | 0x520E | 1 | Exhaust brake switch |  | 0x00/0x01 Off/On |
|  | 0x520F | 2 | Engine reference torque |  |  |
|  | 0x5210 | 1 | Friction torque of auxiliary engine of sanitation vehicle (special) | % | Accuracy: 1. Offset: -125. Value range: -125%~+125% |
|  | 0x5211 | 1 | Temperature of auxiliary engine coolant for sanitation vehicles (special) | ℃ | Accuracy: 1 ℃ Offset: -40.0 ℃ Range: -40.0 ℃~+210 ℃ |
|  | 0x5212 | 1 | Atmospheric pressure of the auxiliary engine of the sanitation vehicle (special) | kPa | Accuracy: 1 Offset: 0 Range: 0~125kpa |
|  | 0x5213 | 1 | Secondary engine load of sanitation vehicle (special) | % | Accuracy: 1 Offset: 0 Value range: 0 to 250% |
|  | 0x5214 | 2 | Secondary engine speed of sanitation vehicle (special) | rpm | Accuracy: 1 Offset: 0 Range: 0~8000 |
|  | 0x5215 | 1 | Percentage of torque of auxiliary engine of sanitation vehicle (special) | % | Accuracy: 1. Offset: -125. Value range: -125%~+125% |
|  | 0x5216 | 1 | Threshold for torque percentage of auxiliary engine of sanitation vehicle (special) | % | Accuracy: 1. Offset: -125. Value range: -125%~+125% |

Appendix_New Energy Vehicle Data Flow

| Function ID field | Function ID[2] | Length[1] | Function | unit | describe |
| --- | --- | --- | --- | --- | --- |
| New energy vehicle data items [0x7001-0x7FFF] | 0x7001 | 4 | Range | 0.1km | Display value is uploaded value/10 |
|  | 0x7002 | 1 | Remaining power (SOC) | % | 0% - 100% |
|  | 0x7003 | 1 | Speed | Km/h | 0 - 240 |
|  | 0x7004 | 1 | Charging status |  | 0x0: Initial value 0x1: Not charging 0x2: AC charging 0x3: DC charging 0x4: Charging completed 0x5: Driving and Charging 0x6: Parking and charging 0x7: Invalid value |
|  | 0x7005 | 1 | Charging station status |  | 0x01: Insert 0x00 : Not inserted |
|  | 0x7006 | 2 | Power battery charging and discharging current | 0.01A | 0x0 -0xFFFF​ Offset -32767 Positive discharge Negative charge |
|  | 0x7007 | 2 | Maximum voltage of single cell | 0.01V​​ | 0x0 -0xFFFF / 100 |
|  | 0x7008 | 2 | Minimum voltage of single cell | 0.01V​​ | 0x0 -0xFFFF / 100 |
|  | 0x7009 | 2 | Current speed of the drive motor | R pm | Offset -32767 For positive motor, the motor rotates forward Negative motor reverse |
|  | 0x700a | 2 | Current torque of driving motor | Nm |  |
|  | 0x700b | 1 | Current temperature of the drive motor | C | Upload value minus 40 |
|  | 0x700c | 2 | DC bus voltage, total voltage | 0.1V | 0x0 -0xFFFF / 10 |
|  | 0x700d | 2 | DC bus current, total current | 0.01A | Offset - 500A Discharging is positive, charging is negative 0x0-0xFFFF/100-500 |
|  | 0x700e | 2 | Power battery available energy | 0.01K wh | 0x0 -0xFFFF​ |
|  | 0x700f | 2 | Power battery total voltage | 0.01 V | 0x0 -0xFFFF​ |
|  | 0x7021 | 2 | No. 1 single battery voltage | 0.01V |  |
|  | 0x7022 | 2 | 2 single battery voltage | 0.01V |  |
|  | 0x7023 | 2 | 3 single battery voltage | 0.01V |  |
|  | 0x7024 | 2 | 4 single battery voltage | 0.01V |  |
|  | 0x7025 | 2 | 5 single battery voltage | 0.01V |  |
|  | 0x7026 | 2 | 6 single battery voltage | 0.01V |  |
|  | 0x7027 | 2 | 7 single battery voltage | 0.01V |  |
|  | 0x7028 | 2 | 8 -cell battery voltage | 0.01V |  |
|  | 0x7029 | 2 | 9 single battery voltage | 0.01V |  |
|  | 0x702A | 1 | 1 AA battery voltage | 0.01V |  |
|  | 0x702B | 1 | BMS heartbeat information |  | 0-255 loop count |
|  | 0x702C | 1 | Highest voltage battery cell code |  |  |
|  | 0x702D | 1 | Lowest voltage battery cell code |  |  |
|  | 0x702E | 1 | Total number of single cells |  |  |
|  | 0x702F | 1 | Total number of temperature probes |  |  |
|  | 0x7030 | 1 | Maximum temperature value | C | Upload value minus 40 |
|  | 0x7031 | 1 | Maximum temperature probe monomer code |  |  |
|  | 0x7032 | 1 | Minimum temperature value | C | Upload value minus 40 |
|  | 0x7033 | 1 | Minimum temperature probe monomer code |  |  |
|  | 0x7034 | 4 | Alarm information |  |  |
|  | 0x7035 | 1 | 1st probe temperature | C | Upload value minus 40 |
|  | 0x7036 | 1 | The second probe temperature | C | Upload value minus 40 |
|  | 0x7037 | 1 | The third probe temperature | C | Upload value minus 40 |
|  | 0x7038 | 1 | 4th probe temperature | C | Upload value minus 40 |
|  | 0x7039 | 1 | 5th probe temperature | C | Upload value minus 40 |
|  | 0x703A | 1 | The 6th probe temperature | C | Upload value minus 40 |
|  | 0x703B | 1 | 7th probe temperature | C | Upload value minus 40 |
|  | 0x703C | 1 | 8th probe temperature | C | Upload value minus 40 |
|  | 0x703D | 1 | 9th probe temperature | C | Upload value minus 40 |
|  | 0x703E | 1 | 10th probe temperature | C | Upload value minus 40 |
|  | 0x703F | 1 | Current battery temperature | ℃ | Upload value minus 40 |
|  | 0x7040 | 1 | Vehicle status |  | 0-Start after flameout |
|  | 0x7041 | 2 | Insulation resistance |  | 0x0 -0xFFFF​ |
|  | 0x7042 | 1 | Battery health status |  | 0-100 |
|  | 0x7043 | 2 | Maximum single cell voltage | 0.01V | 0x0 -0xFFFF /100 |
|  | 0x7044 | 2 | Minimum single cell voltage | 0.01V | 0x0 -0xFFFF /100 |
|  | 0x7045 | 2 | Unit pressure difference | 0.01V | 0x0 -0xFFFF /100 |
|  | 0x7043 | 2 | Maximum single cell voltage | 0.001V | 0x0 -0xFFFF /1000 |
|  | 0x7044 | 2 | Minimum single cell voltage | 0.001V | 0x0 -0xFFFF /1000 |
|  | 0x7045 | 2 | Unit pressure difference | 0.001V | 0x0 -0xFFFF /1000 |
|  | 0x7046 | 1 | Power level |  | 0-1-2 Need to confirm whether it is off/on/start What is the difference from the vehicle status above? Or the vehicle gear? |
|  | 0x7047 | 4 | BMS status | u8 | BMS status： 0 "Init" 1 "Standby" 2 "PreCharge" 3 "Run" 4 "AC Charge" 5 "DC Charge" 6 "PowerDown" 7 "Error" |
|  |  |  |  | u8 | BMS lock status: 0 "UnLocked" 1 "Locked" |
|  |  |  |  | u8 | reserve |
|  |  |  |  | u8 | reserve |
|  | 0x7048 | 2 | average voltage | 0.001V | 0x0-0xFFFF/1000 |
|  | 0x7049 | 1 | Average temperature value | °C | Subtract 40 from the uploaded value |
|  | 0x704A | 1 | Accelerator pedal position | % | Accuracy: 1 Offset: 0 Range: 0%~100% |
|  | 0x704B | 2 | Battery pack capacity |  | Accuracy: 0.1, offset: 0 |
|  | 0x704C | 2 | Remaining power | KwH | Accuracy: 0.01, offset: 0 |

Schedule Extending peripheral data flow

| Function ID field | Function ID[2] | Length[1] | Function | unit | describe |
| --- | --- | --- | --- | --- | --- |
| Peripheral data items 0x3001-0x4FFF | 0x3001 | 1 | Forward and reverse state |  | 0x00 (stop) 0x01 (Forward) 0x02 (reverse) |
|  | 0x3002 | 2 | Probe temperature (1) | 0.1℃ | Starting temperature -40.0℃, upload value minus 40 |
|  | 0x3003 | 2 | Probe temperature (2) | 0.1℃ | Starting temperature -40.0℃, upload value minus 40 |
|  | 0x3004 | 2 | Probe temperature (3) | 0.1℃ | Starting temperature -40.0℃, upload value minus 40 |
|  | 0x3005 | 2 | Probe temperature (4) | 0.1℃ | Starting temperature -40.0℃, upload value minus 40 |
|  | 0x3006 | N | Tire pressure data |  | See tire pressure data table |
|  | 0x3007 | N | Bracelet data pack |  | See bracelet data package (not available yet) |
|  | 0x3008 | 25 | H600 video status information |  | See H600 video status information table |
|  | 0x3009 | 11 | H600 input signal |  | See H600 input signal table |
|  | 0x300A | N | Load cell data package |  | See load cell data sheet |
|  | 0x300B | N | External oil sense data |  | See external oil sensor data sheet |
|  | 0x300C | N | Fire truck 6-channel data acquisition (special) | % | See Fire Truck 6-Channel Data Collection Data Table |
|  | 0x300D | N | Temperature sensor data |  | Thailand Project: 2 bytes: temperature, accuracy 0.1℃ Other projects: 12 bytes: temperature, precision 0.1 34 bytes: humidity, precision 0.1 56 bytes: voltage, precision 0.01 78 bytes; disassembly status and signal strength, the high byte is the disassembly status, 0xFF, not disassembled; 0x00, disassembled; the low byte is the signal strength, a signed number, the reading is the signal strength dBm |
|  | 0x300E | N | High-speed rail tire pressure data |  | See attached table_Extended peripheral data: High-speed rail tire pressure data table |
|  | 0x3013 | 8 | G-sensor value |  | Total value ,x,y,z.every 2 bytes |
|  | 0x3014 | 8 | Input and output status |  | Input and Output status Table |
|  | 0x3015 | 2 | Analog signal input 1 | 0.1 | V |
|  | 0x3016 | 2 | Analog signal input 2 | 0.1 | V |
|  | 0x3017 | N | Driver swipe card data pack( Track 3) |  | Track 3 information |

Appendix_Alarm command ID and description items

| Function ID field | Function ID[2] | Length[1] | Function | illustrate |
| --- | --- | --- | --- | --- |
| 0x0001-0x0500 | 0x0001 | 0 | Ignition report | The above data cannot be reported at the same time. Only one alarm can be reported |
|  | 0x0002 | 0 | Flame out report |  |
|  | 0x0003 | 0 | Defense reporting |  |
|  | 0x0004 | 0 | Disarm report |  |
|  | 0x0005 | 0 | Door open |  |
|  | 0x0006 | 0 | Door closed |  |
|  | 0x000 7 | 0 | System Startup |  |
|  | 0x0101 | 0 | Tow truck alarm |  |
|  | 0x0102 | 0 | Positioning too long alarm |  |
|  | 0x0103 | 0 | Terminal unplug alarm (main power failure) |  |
|  | 0x0104 | 0 | Terminal insertion alarm (main power on) |  |
|  | 0x0105 | 0 | Low voltage alarm |  |
|  | 0x0106 | X | Idle too long alarm |  |
|  | 0x0107 | X | Overspeed alarm |  |
|  | 0x0108 | X | Fatigue driving alarm |  |
|  | 0x0109 | X | Water temperature alarm |  |
|  | 0x010A | 0 | High-speed neutral coasting alarm |  |
|  | 0x010B | 0 | Fuel consumption does not support alarm |  |
|  | 0x010C | 0 | OBD does not support alarm |  |
|  | 0x010D | 0 | Low water temperature and high speed |  |
|  | 0x010E | 0 | Bus not sleeping alarm |  |
|  | 0x010F | 0 | Illegal door opening |  |
|  | 0x0110 | 0 | Illegal ignition |  |
|  | 0x0111 | 0 | Rapid acceleration alarm |  |
|  | 0x0112 | 0 | Rapid deceleration alarm |  |
|  | 0x0113 | 0 | Sharp turn alarm |  |
|  | 0x0114 | 0 | Collision Alarm |  |
|  | 0x0115 | 0 | Abnormal vibration alarm |  |
|  | 0x0201 | 0 | GPS module fault alarm |  |
|  | 0x0202 | 0 | FLASH fault alarm |  |
|  | 0x0203 | 0 | CAN module fault alarm |  |
|  | 0x0204 | 0 | 3D sensor fault alarm |  |
|  | 0x0205 | 0 | RTC module fault alarm |  |
|  | 0x0206 | 0 | Temperature sensor fault alarm |  |
|  | 0x0301 | 0 | Reminder for security glass not closed |  |
|  | 0x0302 | 0 | Unsuccessful lock reminder |  |
|  | 0x0303 | 0 | Timeout without defense reminder |  |
|  | 0x0401 | 0 | Emergency Braking | The current speed is greater than a certain speed, and the difference between the vehicle's speed in the next second and the speed in the previous second exceeds the threshold, triggering the emergency brake alarm; parameter settings see 8103 Speed difference threshold: 9km/h Speed greater than a certain speed: 0km/h |
|  | 0x0402 | 0 | Emergency Braking | If the difference between the vehicle's speed in the next second and the speed in the previous second exceeds the threshold, the emergency brake alarm is triggered; see 8103 for parameter settings Speed difference threshold: 18km/h |
|  | 0x0403 | 0 | Overspeed | When the speed is greater than the set threshold, this alarm is triggered. For parameter settings, see 8103 Engine speed threshold: 2400 rpm |
|  | 0x0404 | 0 | PTO Idle | In idle condition, if the speed is greater than the specified threshold, this alarm is triggered. For parameter setting, see 8103 Engine speed threshold: 1000 rpm |
|  | 0x0405 | 0 | Unplug the OBD connector (4-5) |  |
|  | 0x0406 | 0 | OBD head (4-5) inserted |  |
|  | 0x0407 | 0 | Host removal alarm (probe) |  |
|  | 0x0408 | 0 | Main box is opened alarm (light sensor) |  |
|  | 0x0409 | 0 | New energy charging status changes | Used to update the charging status of energy vehicles in real time |
|  | 0x040A | 0 | Low battery alarm | Triggered when the battery level falls below a certain threshold |
|  | 0x040B | 0 | Unplug the OBD1 connector (6-14) |  |
|  | 0x040C | 0 | Unplug the OBD2 connector (1-9) |  |
|  | 0x040D | 0 | Unplug the OBD2 connector (3-11) |  |
|  | 0x040E | 0 | Unplug the OBD2 connector (11-12) |  |
|  | 0x040F | 0 | Very low voltage alarm |  |
|  | 0x0410 | 0 | Low oil level alarm | 0x00: Alarm cleared; 0x01: Alarm triggered; The terminal can use the low oil pressure field to report a low oil level alarm |

Appendix_Base station data flow

| content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- |
| time bcd [ 6] | 6 | byte | Trigger time YY-MM-DD-hh-mm-ss (GMT+8 time) East 8 time zone, BCD code |
| mcc | n | string | The country code of the mobile user , such as: 460 |
| , | 1 | byte | English half- width comma separator |
| mnc | n | string | Mobile network number, China Mobile: 0; China Unicom: 1 |
| , | 1 | byte | English half- width comma separator |
| base num | 1 | byte | Number of cell information , 0-9 |
| , | 1 | byte | English half- width comma separator |
| lac [1] | n | string | Location area code, value range: 0-65535 |
| , | 1 | byte | English half- width comma separator |
| cellid [1] | n | string | Base station cell number, value range: 0-65535, 0-268435455, of which 0, 65535, 268435455 are not used. When the cell number is greater than 65535, it is a 3G base station. |
| , | 1 | byte | English half- width comma separator |
| signal [1] | n | string | Signal strength , 1-31 |
| ...... |  |  | when base num is greater than 1. LAC[2], Cellid [2], Signal [2], separated by commas The format is as shown in the gray part above, which accumulates multiple cell information in sequence. |

Appendix_Basic data items: Accelerometer

| Overall length | Byte sequence | content | byte | type | unit | illustrate |
| --- | --- | --- | --- | --- | --- | --- |
| N bytes | 0 | Number of collection points in the last second | 2 | u16 |  | The default is 4 points per second. |
|  | 2 | The interval of the sampling points within the last 1 second | 2 | u16 | millisecond |  |
|  | 4 | Acceleration Mean 1 | 2 | u16 | mg | The average acceleration of the first acquisition point |
|  | 6 | Acceleration Mean 2 | 2 | u16 | mg | The average value of the acceleration at the second acquisition point |
|  | 8 | Acceleration Mean 3 | 2 | u16 | mg | The average acceleration of the third acquisition point |
|  | 10 | Acceleration Mean 4 | 2 | u16 | mg | The average acceleration of the fourth acquisition point |
|  | N | Acceleration Mean N | 2 | u16 | mg | The average acceleration value of the Nth acquisition point |
|  | N*2+2 | Acceleration Total Max | 2 | u16 | mg | Maximum acceleration value within 1 second |

Appendix_Basic data items: Total mileage format table

| project | Byte sequence | length | Algorithm Index | Algorithm Name |
| --- | --- | --- | --- | --- |
| Mileage Type | 0 | 1 | 0x01 | GPS total mileage (accumulated) |
|  |  |  | 0x02 | Other 1 [J1939 Mileage Algorithm 1] |
|  |  |  | 0x03 | Other 2 [J1939 Mileage Algorithm 2] |
|  |  |  | 0x04 | Other 3 [J1939 Mileage Algorithm 3] |
|  |  |  | 0x05 | Other 4 [J1939 Mileage Algorithm 4] |
|  |  |  | 0x06 | Others 5 [J1939 Mileage Algorithm 5] |
|  |  |  | 0x07 | OBD meter mileage |
|  |  |  | 0x08 | OBD speed mileage |
|  |  |  | 0x09 | Others 6 [J1939 Mileage Algorithm 6] |
|  |  |  | 0x0A | Others 7 [J1939 Mileage Algorithm 7] |
|  |  |  | 0x0B | Others 8 [J1939 Mileage Algorithm 8] |
|  |  |  | 0x0C | Others 9 [J1939 Mileage Algorithm 9] |
| Total mileage | 1 | 4 | Unit: meter |  |

Appendix_Basic data items: Accumulated mileage 2 format table

| project | Byte sequence | length | Algorithm Index | Algorithm Name |
| --- | --- | --- | --- | --- |
| Accumulation Type | 0 | 1 | 0x01 | GPS speed accumulation |
|  |  |  | 0x02 | OBD speed accumulation |
|  |  |  | 0x03 | OBD instrument accumulation |
| Accumulated mileage 2 | 1 | 4 | Unit: meter |  |

Appendix_Basic data items: Total fuel / Electricity consumption format table

| project | Byte sequence | length | Fuel consumption type | Algorithm Name |
| --- | --- | --- | --- | --- |
| Fuel / Electricity consumption type | 0 | 1 | 0x01 | J1939 fuel consumption algorithm 1 |
|  |  |  | 0x02 | J1939 fuel consumption algorithm 2 |
|  |  |  | 0x03 | J1939 fuel consumption algorithm 3 |
|  |  |  | 0x04 | J1939 fuel consumption algorithm 4 |
|  |  |  | 0x05 | J1939 fuel consumption algorithm 5 |
|  |  |  | 0x0B | OBD fuel consumption algorithm 1 |
|  |  |  | 0x0C | OBD fuel consumption algorithm 2 |
|  |  |  | 0x0D | OBD fuel consumption algorithm 3 |
|  |  |  | 0x0E | OBD fuel consumption algorithm 4 |
|  |  |  | 0x10 | OBD fuel consumption algorithm 5 |
|  |  |  | 0xA0 | Electricity consumption algorithm 1 |
| Total fuel / Electricity consumption | 1 | 4 | Unit: ML (accuracy 1) / KWH (accuracy 0.01) |  |

Appendix_Basic data items: Accelerometer

| Overall length | Byte sequence | content | byte | type | unit | illustrate |
| --- | --- | --- | --- | --- | --- | --- |
| N bytes | 0 | Number of collection points | 2 | u16 |  |  |
|  | 2 | Collection point interval | 2 | u16 | millisecond |  |
|  | 4 | Acceleration Mean 1 | 2 | u16 | mg | The average acceleration of the first acquisition point |
|  | 6 | Acceleration Mean 2 | 2 | u16 | mg | The average value of the acceleration at the second acquisition point |
|  | 8 | Acceleration Mean 3 | 2 | u16 | mg | The average acceleration of the third acquisition point |
|  | 10 | Acceleration Mean 4 | 2 | u16 | mg | The average acceleration of the fourth acquisition point |
|  | N | Acceleration Mean N | 2 | u16 | mg | The average acceleration value of the Nth acquisition point |
|  | N+2 | Acceleration Total Max | 2 | u16 | mg | Maximum acceleration value during acquisition time |

Appendix_Basic data items: Protocol type table

|  | Numeric | Protocol Type |
| --- | --- | --- |
| OBD protocol type table | 0X11 | CAN 11_500 |
|  | 0X12 | CAN 11_250 |
|  | 0X13 | CAN 29_500_EX |
|  | 0X14 | CAN 29_250_EX |
|  | 0X20 | KWP2000 |
|  | 0X30 | KWP2000M |
|  | 0X40 | ISO9141 |
|  | 0X50 | VPW |
|  | 0X60 | PWM |
|  | 0X70 | PRIVATE |
|  | 0XF0 | J1939 |

Appendix_Basic data items: Vehicle status table

| Segment Sequence | Subsequence | content | Word count | data type | Accuracy | describe |
| --- | --- | --- | --- | --- | --- | --- |
| Status Mask | 1 | Status Mask | 10 | u8 |  | Car status mask |
|  |  |  |  |  |  | Indicates whether the following 10 types of car status are supported or not |
| Status Field | 1 | Security Status | 1 | u8 |  | Bit0 1/0 ON/OFF ACC status |
|  |  |  |  |  |  | Bit1 1/0 Arm/Disarm Arm/Disarm status |
|  |  |  |  |  |  | Bit2 1/0 Press/release the foot brake |
|  |  |  |  |  |  | Bit3 1/0 Press/release the accelerator |
|  |  |  |  |  |  | Bit4 1/0 Pull up/release the handbrake |
|  |  |  |  |  |  | Bit5 1/0 Insert/release the main seat belt |
|  |  |  |  |  |  | Bit6 1/0 Insert/release the auxiliary seat belt |
|  |  |  |  |  |  | Bit7 1/0 ON/OFF Engine status |
|  | 2 | Door status | 1 | u8 |  | Bit0 1/0 Open/close left front door LF |
|  |  |  |  |  |  | Bit1 1/0 Turn on/off the right front door RF |
|  |  |  |  |  |  | Bit2 1/0 Open/close left rear door LB |
|  |  |  |  |  |  | Bit3 1/0 Open/close right rear door RB |
|  |  |  |  |  |  | Bit4 1/0 Open/close trunk |
|  |  |  |  |  |  | Bit5 1/0 Open/close engine hood |
|  |  |  |  |  |  | Bit6 1/0 Reserved |
|  |  |  |  |  |  | Bit7 1/0 Reserved |
|  | 3 | Lock Status | 1 | u8 |  | Bit0 1/0 Lock/unlock left front lock LF |
|  |  |  |  |  |  | Bit1 1/0 Lock/unlock right front lock RF |
|  |  |  |  |  |  | Bit2 1/0 Lock/unlock left rear lock LB |
|  |  |  |  |  |  | Bit3 1/0 Lock/unlock right rear lock RB |
|  |  |  |  |  |  | Bit4 1/0 Reserved |
|  |  |  |  |  |  | Bit5 1/0 Reserved |
|  |  |  |  |  |  | Bit6 1/0 Reserved |
|  |  |  |  |  |  | Bit7 1/0 Reserved |
|  | 4 | Window Status | 1 | u8 |  | Bit0 1/0 Open/close left front window LF |
|  |  |  |  |  |  | Bit1 1/0 Turn on/off the right front window RF |
|  |  |  |  |  |  | Bit2 1/0 Open/close left rear window LB |
|  |  |  |  |  |  | Bit3 1/0 Open/close right rear window RB |
|  |  |  |  |  |  | Bit4 1/0 Sunroof on/off switch |
|  |  |  |  |  |  | Bit5 1/0 Turn on/off the left turn signal |
|  |  |  |  |  |  | Bit6 1/0 Turn on/off the right turn signal |
|  |  |  |  |  |  | Bit7 1/0 Turn on/off the reading light |
|  | 5 | Lighting status 1 | 1 | u8 |  | Bit0 1/0 Turn on/off low beam |
|  |  |  |  |  |  | Bit1 1/0 Turn on/off high beam |
|  |  |  |  |  |  | Bit2 1/0 Turn on/off the front fog lights |
|  |  |  |  |  |  | Bit3 1/0 Turn on/off rear fog lights |
|  |  |  |  |  |  | Bit4 1/0 Turn on/off the hazard lights |
|  |  |  |  |  |  | Bit5 1/0 Turn on/off the reversing light |
|  |  |  |  |  |  | Bit6 1/0 Turn on/off AUTO light |
|  |  |  |  |  |  | Bit7 1/0 Turn on/off the width indicator light |
|  | 6 | Switch state A | 1 | u8 |  | Bit0 1/0 ON/OFF Oil alarm |
|  |  |  |  |  |  | Bit1 1/0 ON/OFF Fuel alarm |
|  |  |  |  |  |  | Bit2 1/0 Turn on/off the wiper |
|  |  |  |  |  |  | Bit3 1/0 Turn on/off the speaker |
|  |  |  |  |  |  | Bit4 1/0 Turn on/off the air conditioner |
|  |  |  |  |  |  | Bit5 1/0 Rearview mirror on/off status |
|  |  |  |  |  |  | Bit6 1/0 Reserved |
|  |  |  |  |  |  | Bit7 1/0 Reserved |
|  | 7 | Switch state B | 1 | u8 |  | Bit0-Bit3 Reserved |
|  |  |  |  |  |  | Bit4-BIT7 gear position |
|  |  |  |  |  |  | ==0 P ==1 R ==2 N ==3 D ==4 1 |
|  |  |  |  |  |  | ==5 2 ==6 3 ==7 4 ==8 5 ==9 6 |
|  |  |  |  |  |  | ==10 M ==11 S ==12 B ==13 L ==15 does not exist |
|  | 8 | Switch state C | 1 | u8 |  | Bit0 1/0 Power supply/outage Power supply status of the key |
|  |  |  |  |  |  | Bit1 1/0 Existence/Non existence Check if the key is present |
|  |  |  |  |  |  | Bit2 1/0 Reserved |
|  |  |  |  |  |  | Bit3 1/0 Reserved |
|  |  |  |  |  |  | Bit4 1/0 Reserved |
|  |  |  |  |  |  | Bit5 1/0 Reserved |
|  |  |  |  |  |  | Bit6 1/0 Reserved |
|  |  |  |  |  |  | Bit7 1/0 Reserved |
|  |  |  |  |  |  | Bit2 1/0 Reserved |
|  | 9 | Reserve | 1 | u8 |  | Reserve |
|  | 10 | Reserve | 1 | u8 |  | Reserve |

Appendix_Alarm Description :  Idle alarm description

| Byte sequence | project | length | unit | describe |
| --- | --- | --- | --- | --- |
| 0 | Idle alarm properties | 1 |  | 0x00 : Alarm cleared; the following data items are included 0x01: Alarm triggered; no content item below the data |
| 1 | Alarm duration | 2 | Second |  |
| 3 | Idle fuel consumption | 2 | M L |  |
| 5 | Maximum idle speed | 2 | RPM |  |
| 7 | Minimum idle speed | 2 | RPM |  |

Appendix_Alarm Description :  Overspeed alarm description

| Byte sequence | project | length | unit | describe |
| --- | --- | --- | --- | --- |
| 0 | Speeding alarm properties | 1 |  | 0x00 : Alarm cleared; the following data items are included 0x01: Alarm triggered; no content item below the data |
| 1 | Alarm duration | 2 | Second |  |
| 3 | Exceeding maximum speed | 2 | 0.1KM/H |  |
| 5 | Average speed | 2 | 0.1KM/H |  |
| 7 | Speeding distance | 2 | rice |  |

Appendix_Alarm Description :  Fatigue driving alarm description

| Byte sequence | project | length | unit | describe |
| --- | --- | --- | --- | --- |
| 0 | Fatigue alarm properties | 1 |  | 0x00 : Alarm cleared; the following data items are included 0x01: Alarm triggered; no content item below the data |
| 1 | Alarm duration | 2 | Second |  |

Appendix_Alarm Description :  Water temperature too high alarm description

| Byte sequence | project | length | unit | describe |
| --- | --- | --- | --- | --- |
| 0 | Water temperature alarm properties | 1 |  | 0x00 : Alarm cleared; the following data items are included 0x01: Alarm triggered; no content item below the data |
| 1 | Alarm duration | 4 | Second |  |
| 5 | Maximum temperature | 2 | 0.1 degree |  |
| 7 | Average temperature | 2 | 0.1 degree |  |

Appendix_Extended peripheral data : H600 video status information table

| Bit | definition | illustrate |
| --- | --- | --- |
| 1 | Total number of channels | Number of camera channels (1-4) |
| 2 | Intercom Request | 0: No intercom request 1: The device is initiating an intercom request |
| 3 | Live Video | 0: Not connected, non-zero: Video is being transmitted on the Internet bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| 4 | Intercom status | 0: Not started, 1: Intercom in progress |
| 5 | Video playback | 0: Not started, non-zero: Remote playback channel bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| 6 | SD1 status | 0: does not exist, 1: normal, 0xff: disk error |
| 7 | SD2 status | 0: does not exist, 1: normal, 0xff: disk error |
| 8 | HDD Status | 0: does not exist, 1: normal, 0xff: disk error |
| 9 | USB disk status | 0: does not exist, 1: normal, 0xff: disk error |
| 10 | EMMC status | 0: does not exist, 1: normal, 0xff: disk error |
| 11 | Working disk | 0xff: No working disk, 0: SD1 is the working disk, 1: SD2 is the working disk, 2: Hard disk is the working disk |
| 12 | Video Status | 0: Video is normal, non-zero: Channel video loss abnormality bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| 13 | Video Blocking | 0: Video is normal, non-zero: Channel video is blocked abnormally bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| 14 | Channel Video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 15 | C hanne2 video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 16 | C hanne3 video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 17 | China4 video​ | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 18 | C hanne5 video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 19 | China6 Video​ | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| 20 | C hanne7 video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| twenty one | C hanne8 video | 0: No recording, 1: Scheduled recording, 2: Manual recording, 3: Alarm recording |
| twenty two | Disaster recovery video | 0: Video not recorded, non-zero: Channel video recording bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| twenty three | emmc video | 0: Video not recorded, non-zero: Channel video recording bit0 first channel, bit1 second channel, bit2 third channel, bit3 fourth channel |
| twenty four | Authorization Status | 0: Unauthorized, 1: Authorized |
| 25 | AV Output | The high 4 bits indicate the number of images, and the low 4 bits indicate the zoom sequence number. 0x11-0x16 single screen 0x20:2 screen, 0x40:4 screen, 0x60:6 screen, 0x90:9 screen For single screen, 0x11: channel 1 zoom 0x12: channel 2 zoom 0x16: Channel 6 amplification |

Appendix_Extended peripheral data : H600 input signal

| Bit | definition | illustrate |
| --- | --- | --- |
| 0 | Signal 1 | Brake signal (high trigger) 1: Triggered 0: Not triggered |
| 1 | Signal 2 | Low beam signal (high trigger) 1: Triggered 0: Not triggered |
| 2 | Signal 3 | High beam signal (high trigger) 1: trigger 0: not trigger |
| 3 | Signal 4 | Left turn signal (high trigger) 1: Triggered 0: Not triggered |
| 4 | Signal 5 | Right turn signal (high trigger) 1: Triggered 0: Not triggered |
| 5 | Signal 6 | Custom high 1 signal (high trigger) 1: trigger 0: not triggered |
| 6 | Signal 7 | Custom high 2 signal (high trigger) 1: trigger 0: not triggered |
| 7 | Signal 8 | Hijack alarm signal (low trigger) 1: trigger 0: not trigger |
| 8 | Signal 9 | Gate signal (low trigger) 1: Trigger 0: Not triggered |
| 9 | Signal 10 | Custom low 1 signal (low trigger) 1: trigger 0: not triggered |
| 10 | Signal 11 | Custom low 2 signal (low trigger) 1: trigger 0: not triggered |

Appendix_Extended peripheral data: Tire pressure data table

| Overall length | Byte sequence | type | length | content | describe |
| --- | --- | --- | --- | --- | --- |
| 4+4*N | 0 | u32 | 4 | Tire Mask | BIT31-BIT0 High bit first, low bit last BIT31: Tire No. 1 (if 1, there is a tire pressure byte, otherwise it is empty) BIT30: Tire No. 2 (if 1, there is a tire pressure byte, otherwise it is empty) .................. BIT0: Tire No. 32 (if 1, there is a tire pressure byte, otherwise it is empty) |
|  | 4 | u16 | 2 | Tire Pressure No. X | Unit 1 Kpa |
|  | 6 | u8 | 1 | Tire temperature of tire No. X | Unit 1 C Display value is the uploaded value minus 40 C |
|  | 7 | u8 | 1 | Tire No. X Status | BYTE BIT7: Quick Leak BIT6: Slow leakage BIT5: Low power BIT4: High temperature BIT3: High Voltage BIT2: Low voltage Other reservations |
|  | ... | ... | ... | ... | ... |
|  | N | u16 | 2 | Tire pressure of tire X+N | Unit 1 Kpa |
|  | N+2 | u8 | 1 | Temperature of tire No. X+N | Unit 1 C Display value is the uploaded value minus 40 C |
|  | N+3 | u8 | 1 | Tire status X+N | BYTE BIT7: Quick Leak BIT6: Slow leakage BIT5: Low power BIT4: High temperature BIT3: High Voltage BIT2: Low voltage Other reservations |

Note: Only tires with the tire mask set will have tire pressure bytes following them

Example: 0x80000000  When mask Only the tire pressure of tire number one is followed.

Example: 0x88000000   0x0082 0x10 0x00 0x0096 0x11 0x80

represent

Tire pressure of No. 1 tire is 130kpa, tire temperature is 16C, status: normal;

Tire No. 5 tire pressure 150kpa, tire temperature 17C, status: leaking

Appendix_Load Cell Data Sheet

| Fields | Data Types | Fields | Description and requirements |
| --- | --- | --- | --- |
| 0 | BYTE | type | 1: Unit - tons; 2: Unit - kilograms |
| 1 | Word​ | Rated load | Rated load, no fill 0 |
| 2 | Word | Current load | Current load |
| 3 | BYTE | Primitive Data Types | If there is no original data, this and the following fields may not be present. 1-Jiangsu Super Control |
| 4 | BYTE[N] | Raw data |  |

Appendix_External oil sensor data sheet

| Overall length | Byte sequence | type | length | content | describe |
| --- | --- | --- | --- | --- | --- |
| 3+5*N | 0 | u8 | 1 | Valid flag | 0: Invalid 1: Valid (Oil sense online) |
|  | 1 | u8 | 1 | Oily type | 0: Xide ultrasonic oil sensor; 1: Omnicomm oil sensor; 2: differential pressure sensor |
|  | 2 | u8 | 1 | Oily Mask | BIT7-BIT0 High bit first, low bit last BIT6: No. 1 oil sense (if 1, there is an oil sense byte later, otherwise it is empty) BIT5: No. 2 oil sense (if 1, there is an oil sense byte later, otherwise it is empty) .................. BIT0: No. 8 oil sense (if 1, there is an oil sense byte later, otherwise it is empty) |
|  | 3 | u8 | 1 | No. 1 Oily Unit | 1: 0.1mm 2: 0.1% 3: 0.1ml |
|  | 3+1 | u32 | 4 | No. 1 oiliness value | Actual value |
|  | … | … | … | … | … |
|  | 3+（N-1）*5 | u8 | 1 | No. 1 Oily Unit | 1: 0.1mm 2: 0.1% 3: 0.1ml |
|  | 3+（N-1）*5+1 | u32 | 4 | N oiliness value | Actual value |

Appendix_Fire truck 6-way data collection data table

| Overall length | Byte sequence | type | length | content | describe |
| --- | --- | --- | --- | --- | --- |
| 12 | 0 | WORD | 2 | Fire truck water tank level | Actual value unit 0.01% (uploaded value divided by 100 is xx.xx%) |
|  | 2 | WORD | 2 | Fire truck foam tank level | Actual value unit 0.01% (uploaded value divided by 100 is xx.xx%) |
|  | 4 | WORD | 2 | Reserve |  |
|  | 6 | WORD | 2 | Reserve |  |
|  | 8 | WORD | 2 | Reserve |  |
|  | 10 | WORD | 2 | Reserve |  |

Appendix_Version Information Package

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Terminal software version number | STRING[14] | Software version number: HL M200_ V201 001 HL --------Product Name M200_ ------Terminal name code V20 1-------Software major version number, release version 001 --------Software minor version number, submitted as internal test |
| 14 | Terminal software version date | STRING[10] | Software Date: 2018-11-19 |
| twenty four | CPU ID Number | BYTE[12] |  |
| 36 | GSM TYPE Name | STRING[15] | GSM Model: |
| 51 | GSM IMEI Number | STRING[15] | GSM IMEI Number |
| 66 | SIM card IMSI number | STRING[15] | Terminal SIM card IMSI number |
| 81 | SIM card ICCID | STRING[20] | Terminal SIM card ICCID number |
| 101 | Car Type | WORD | Car model ID |
| 103 | VIN | STRING[17] | Car VIN number |
| 120 | Total mileage | DWORD | Total vehicle mileage or vehicle instrument mileage after the terminal is installed (m) |
| 124 | Total fuel consumption | DWORD | Total fuel consumption of the vehicle after installing the terminal (ml) |

Appendix_Version Information Packet Response

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Current platform time | BYTE[6] | Year, month, day, hour, minute, second (BCD code) Beijing time, Eastern Time Zone 8 For example: 0x19, 0x01, 0x28, 0x18, 0x10, 0x30 19/1/28 18:10:30 Beijing time |
| 6 | Model ID | WORD | If you do not need to set the model, fill in 0 |
| 8 | Displacement | WORD | Unit is milliliter, fill in 0 if no setting is needed |
| 10 | Upgrade | BYTE | 0x55 upgrade , others not upgrade |

Appendix_Text message body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Logo | BYTE | Appendix of text information flag meaning |
| 1 | Text information | STRING | The maximum length is 102 bytes , encoded in GBK |

Appendix_Meaning of text information flags

| Bit | Logo |
| --- | --- |
| 0 | 1 : Urgent ( used for sending text messages ) |
| 1 | reserve |
| 2 | 1 : Terminal display shows |
| 3 | 1 : Terminal TTS playback |
| 4 | 1 : Advertising screen display |
| 5 | 1 : HUD text data transparent transmission |
| 6-7 | reserve |

Appendix_Text message body​

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Logo | BYTE | '0' represents TXT_BG2312 , '1' represents TXT_UNICODE |
| 1 | Glyphs | STRING | The default value is "*prompt*" , which takes up 6 bytes. |
| 7 | Text information | STRING | The maximum length is 102 4 bytes , encoded in GBK |

Appendix_Data Uplink Transparent Transmission Message Body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | Transparent message type | BYTE | Transparent message type definition table |
| 1 | Transparent message content | [N] BYTE | Corresponding message content |

Appendix_Transparent message type definition

| Transparent message type | Transparent message content | Description and requirements |
| --- | --- | --- |
| 0xF1​ | Driving trip data (sent when the engine is turned off) | Driving trip data package |
| 0xF2​ | Fault code data (sent when status changes) | Fault code data package |
| 0xF3​ | Sleep entry (sent when entering sleep mode) | Sleep Entry Packet |
| 0xF4​ | Sleep wake-up (send after exiting sleep mode) | Sleep wake- up packet |
| 0xF5 | Vehicle GPS Lite Data Pack (Truck Version) | Not yet joined |
| 0xF6 | MCU upgrade status feedback package | MCU upgrade status feedback package |
| 0xF7 | Suspected collision alarm description package | Suspected collision alarm description package |
| 0x41 | Serial port 1 transparent transmission (upstream/downstream) | Serial port 1 transmits information transparently |
| 0x42 | Serial port 2 transparent transmission (upstream/downstream) | Serial port 2 transmits information transparently |

Appendix_Driving trip data package F1

| Fields | Data Types | Description and requirements |
| --- | --- | --- |
| Information ID | WORD |  |
| Message length | BYTE |  |
| Information content |  | Driving trip data dynamic information table |

Appendix_Driving trip data dynamic information table

| Information ID | information length | Information content | type | describe |
| --- | --- | --- | --- | --- |
| 0x0001 | 6 | ACC ON Time BCD[6] | u8 | YY-MM-DD-hh-mm-ss (GMT+8 time ) East 8 time zone, BCD code |  |
| 0x0002 | 6 | ACC OFF Time BCD[6] | u8 | YY-MM-DD-hh-mm-ss (GMT+8 time) East 8 time zone, BCD code |
| 0x0003 | 4 | ACC ON Latitude | u32 | Unit : 0.000001 degrees , Bit31=0/1 North latitude/South latitude |
| 0x0004 | 4 | ACC ON Longitude | u32 | Unit: 0.000001 degrees , Bit31=0/1 East longitude /West longitude |
| 0x0005 | 4 | ACC OFF Latitude | u32 | Unit : 0.000001 degrees , Bit31=0/1 North latitude/South latitude |
| 0x0006 | 4 | ACC OFF Longitude | u32 | Unit: 0.000001 degrees , Bit31=0/1 East longitude /West longitude |
| 0x0007 | 2 | Trip Mark | u16 | Drive cycle tags |
| 0x0008 | 1 | Trip Distance Type | u8 | Total mileage for a driving cycle type: 0x01 GPS total mileage (accumulated) 0x02 Other 1 [ J1939 mileage algorithm 1 ] 0x03 Other 2 [ J1939 mileage algorithm 2 ] 0x04 Other 3 [ J1939 mileage algorithm 3 ] 0x05 Other 4 [ J1939 mileage algorithm 4 ] 0x06 Other 5 [ J1939 mileage algorithm 5 ] 0x07 OBD meter mileage 0x08 OBD speed mileage 0x09 Other 6 [ J1939 mileage algorithm 6 ] 0x0A Other 7 [ J1939 mileage algorithm 7 ] 0x0B Other 8 [ J1939 mileage algorithm 8 ] 0x0C Other 9 [ J1939 mileage algorithm 9 ] |
| 0x0009 | 4 | Trip Distance | u32 | Total mileage of a driving cycle , in meters |
| 0x000A | 4 | Trip Fuel Consum | u32 | Total fuel consumption for a driving cycle , in milliliters (ml) |
| 0x000B | 4 | Trip Duration Total | u32 | Total duration of a driving cycle , in seconds |
| 0x000C | 2 | Trip Overspeed Duration | u16 | The cumulative speeding time in a driving cycle , in seconds |
| 0x000D | 2 | Trip Overspd Times | u16 | Number of speeding in a driving cycle , unit: times |
| 0x000E | 1 | Trip Speed Average | u8 | Average speed of a driving cycle , unit KM/H |
| 0x000F | 1 | Trip Speed Maximum | u8 | Maximum vehicle speed in a driving cycle , unit: KM/H |
| 0x0010 | 4 | Trip Idle Duration | u32 | Idle time of a driving cycle , in seconds |
| 0x0011 | 1 | Trip Mask of Braking | u8 | The number of foot brakes in a driving cycle is supported or not , 1 means supported |
| 0x0012 | 2 | Trip Number of Braking | u16 | The total number of foot brakes in a driving cycle , unit: times |
| 0x0013 | 4 | Trip Accelerate times | u32 | Number of rapid accelerations in a driving cycle |
| 0x0014 | 4 | Trip Decelerate Times | u32 | Number of rapid decelerations in a driving cycle |
| 0x0015 | 4 | Trip Sharp turn times | u32 | Number of sharp turns in a driving cycle |
| 0x0016 | 4 | Trip Miles Spd less than 20Km/H | u32 | Mileage at a speed of -20Km/H , unit : m |
| 0x0017 | 4 | Trip Miles Spd between 20-40Km/H | u32 | Mileage at speed of 20-40Km/H , unit : m |
| 0x0018 | 4 | Trip Miles Spd between 40-60Km/H | u32 | Mileage at a speed of 40-60Km/H , unit : m |
| 0x0019 | 4 | Trip Miles Spd between 60-80Km/H | u32 | Mileage at a speed of 60-80Km/H , unit : m |
| 0x001A | 4 | Trip Miles Spd between 80-100Km/H | u32 | Mileage at speed of 80-100Km/H , unit : m |
| 0x001B | 4 | Trip Miles Spd between 100-120Km/H | u32 | Mileage at speed 100-120Km/H , unit : m |
| 0x001C | 4 | Trip Miles Spd Over 120Km/H | u32 | Mileage with speed above 120Km/H , unit : m |
| 0x001 D | 4 | Idle fuel consumption | u32 | Idle fuel consumption value in a trip, unit: ML |

Appendix_Fault code data package F2

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | TIME BCD[ 6 ] | 6 | u8 | YY-MM-DD-hh-mm-ss (GMT+8 time) |
| 6 | latitude | 4 | u32 | Unit : 0.000001 degrees , Bit31=0/1 North latitude/South latitude |
| 10 | longitude | 4 | u32 | Unit: 0.000001 degrees , Bit31=0/1 East longitude /West longitude |
| 14 | Dtc Num | 1 | u8 | 0 means no fault code , non-0 means the number of fault codes |
| 15 | Dtc1 ID | 4 | BYTE | The first fault code ID : 4 bytes |
| 19 | Dtc2 ID | 4 | BYTE | The second fault code ID : 4 bytes |
| twenty three | Dtc3 ID | 4 | BYTE | The third fault code ID : 4 bytes |
| … | … | … | … | … |

Note: A fault code number consists of 4 bytes:

If the protocol type is not 0xF0 ( i.e. not J1939 protocol ), they are system ID, fault byte 1, fault byte 2, and fault byte 3 respectively ;

If the protocol type is OXF0 , the first 3 bytes are the fault code bytes and the fourth byte is the fault code status.

Appendix_Sleep entry data packet F3

| Byte position | content | Number of bytes | Number Type | describe |
| --- | --- | --- | --- | --- |
| 0 | Time BCD[6] | 6 | u8 | Sleep entry time YY-MM-DD-hh-mm-ss (GMT+8 time) East 8 time zone, BCD code |  |

Appendix_Sleep wake - up data packet F4

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | Time BCD[6] | 6 | u8 | Sleep wake-up time YY-MM-DD-hh-mm-ss (GMT+8 time) East 8 time zone, BCD code |
| 6 | Wake Type | 1 | u8 | Heartbeat0X01 ​ CAN1 0X02 Low voltage 0X04 G-SENSOR 0X08 ACC interruption 0X10 GSM 0X20 Voltage threshold reaches 0X40 Voltage fluctuation 0X80 |
| 7 | Bat Vol | 2 | u16 | Bus voltage |
| 9 | Accel Total | 2 | u16 | Vibration acceleration value |

Appendix MCU upgrade status feedback package F6

| Byte position | content | Number of bytes | Number Type | describe |
| --- | --- | --- | --- | --- |
| 0 | Post-upgrade status | 1 | u8 | 0x00 Success 0x01 The software version numbers are the same 0x02 The upgrade parameter format is incorrect 0x03 FTP login timeout 0x04 Download timeout 0x05 File verification error 0x06 File type error 0x07 File does not exist 0x08 Other errors |  |

Appendix_Suspected Collision Alarm Description Package F7

After a collision is triggered, the collision data at fixed time points before and after the collision are collected at a specified frequency, and then reported to the platform through the F7 command

Note: When the device supports F7 collision reporting, the 0x0114 collision command in the 0200 alarm data does not need to be reported, and the F7 command shall prevail.

| Byte position | content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- | --- |
| 0 | TIME BCD[ 6 ] | 6 | unsigned char | YY-MM-DD-hh-mm-ss (GMT+8 time) , the time when the collision occurred |
| 6 | latitude | 4 | unsigned int | Unit: 0.000001 degrees , Bit31 = 0/1 North latitude/South latitude, latitude when collision occurred |
| 10 | longitude | 4 | unsigned int | Unit: 0.000001 degrees , Bit31 = 0/1 East longitude/West longitude, longitude when collision occurred |
| 14 | Collection frequency | 4 | unsigned int | The frequency of collecting raw data ensures that a total of 20 seconds of data before and after the collision can be collected. The default collection frequency is: 500 milliseconds, which can be modified <1>, frequency 1000 milliseconds, total number of acquisitions 20 times, total time 20 seconds <2>, frequency 500 milliseconds, total number of acquisitions 40 times, total time 20 seconds <3>, frequency 250 milliseconds, total number of acquisitions 80 times, total time 20 seconds |
| 18 | Collision Level | 1 | unsigned char | 0x00: Mild level 0x01: Moderate level 0x02: Severity level |
| 19+((N-1)*7)+0 | X-axis acceleration[1] | 2 | signed short int | N=1; unit: mg; range: -32768~32768 |
| 19+((N-1)*7)+2 | Y-axis acceleration[1] | 2 | signed short int | N=1; unit: mg; range -32768~32768 |
| 19+((N-1)*7)+4 | Z-axis acceleration[1] | 2 | signed short int | N=1; unit: mg; range -32768~32768 |
| 19+((N-1)*7)+6 | Speed[1] | 1 | unsigned char | N=1; unit: km/h |
| 19+((N-1)*7)+0 | X-axis acceleration[2] | 2 | signed short int | N=2; unit: mg; range: -32768~32768 |
| 19+((N-1)*7)+2 | Y-axis acceleration[2] | 2 | signed short int | N=2; unit: mg; range: -32768~32768 |
| 19+((N-1)*7)+4 | Z-axis acceleration[2] | 2 | signed short int | N=2; unit: mg; range: -32768~32768 |
| 19+((N-1)*7)+6 | Speed[2] | 1 | unsigned char | N=2; unit: km/h |
| ... | ... | ... | ... | ... |
| ... | ... | ... | ... | ... |
| ... | ... | ... | ... | ... |
| ... | ... | ... | ... | ... |
| 19+((N-1)*7)+0 | X-axis acceleration [N] | 2 | signed short int | N=N; unit: mg; range -32768~32768 |
| 19+((N-1)*7)+2 | Y-axis acceleration [N] | 2 | signed short int | N=N; unit: mg; range -32768~32768 |
| 19+((N-1)*7)+4 | Z-axis acceleration [N] | 2 | signed short int | N=N; unit: mg; range -32768~32768 |
| 19+((N-1)*7)+6 | Speed [N] | 1 | unsigned char | N=N; unit: km/h |

Appendix_ CAN broadcast data flow [Customized functions]

| content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- |
| time | 6 | BCD[6] | YY-MM-DD-hh-mm-ss (GMT+8 equipment reports using Beijing time) |
| Total number of CAN IDs | 2 | Word | The total number of CAN ID data collected by the bus, as shown below, corresponds to how many lists, N |
| CAN List[1] | 12 | BYTE[12] | Total 12 bytes, the first 4 bytes are CAN ID, the last 8 bytes are the corresponding data stream |
| ... |  |  |  |
| CAN List[N] | 12 | BYTE[12] | Total 12 bytes, the first 4 bytes are CAN ID, the last 8 bytes are the corresponding data stream |

Appendix_New Energy Vehicle BMS Data Information Body

| Start Byte | Fields | Data Types | Description and requirements |
| --- | --- | --- | --- |
| 0 | time | BCD[6] | YY-MM-DD-hh-mm-ss (GMT+8 equipment reports using Beijing time) |
| 6 | BMS data content length | WORD |  |
| 8 | BMS data content | nByte | For details, see New Energy Vehicle BMS Data Flow Data format: Data packet contains sub-ID (2 BYTE), length ( 2 BYTE) + data (NBYTE) |

Appendix_New Energy Vehicle BMS Data Flow

| Function ID[2] | Length[2] | Function | unit | describe |
| --- | --- | --- | --- | --- |
| 0x0 001 | N | Single battery pack voltage |  | Single battery pack voltage data table |
| 0x0002 | N | Single battery pack temperature |  | Single battery pack temperature data table |

Appendix_New Energy Vehicle BMS Data Stream : Single Battery Pack Voltage Data Table

| Overall length | Byte sequence | type | content | describe |
| --- | --- | --- | --- | --- |
| 16+2*N | 0 | DWORD | Single battery pack voltage mask 0 | BIT31-BIT0 High bit first, low bit last BIT31: No. 1 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT30: No. 2 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 32 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 4 | DWORD | Single battery pack voltage mask 1 | BIT31-BIT0 High bit first, low bit last BIT31: No. 33 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) BIT30: No. 34 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 64 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 8 | DWORD | Single battery pack voltage mask 2 | BIT31-BIT0 High bit first, low bit last BIT31: No. 65 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT30: No. 66 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 96 single battery pack (if it is 1, there is a single battery pack byte, otherwise it is empty) |
|  | 12 | DWORD | Single battery pack voltage mask 3 | BIT31-BIT0 High bit first, low bit last BIT31: No. 96 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT30: No. 66 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 128 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 16 | DWORD | Single battery pack voltage mask 4 | BIT31-BIT0 High bit first, low bit last BIT31: No.129 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 160 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 20 | DWORD | Single battery pack voltage mask 5 | BIT31-BIT0 High bit first, low bit last BIT31: No.161 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 192 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 24 | DWORD | Single battery pack voltage mask 6 | BIT31-BIT0 High bit first, low bit last BIT31: No.193 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.224 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 28 | DWORD | Single battery pack voltage mask 7 | BIT31-BIT0 High bit first, low bit last BIT31: No.225 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.256 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 32 | DWORD | Single battery pack voltage mask 8 | BIT31-BIT0 High bit first, low bit last BIT31: No.257 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.288 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 36 | DWORD | Single battery pack voltage mask 9 | BIT31-BIT0 High bit first, low bit last BIT31: No.289 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.320 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 40 | DWORD | Single battery pack voltage mask 10 | BIT31-BIT0 High bit first, low bit last BIT31: No.321 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.352 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 44 | DWORD | Single battery pack voltage mask 11 | BIT31-BIT0 High bit first, low bit last BIT31: No.353 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.384 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 48 | WORD | Voltage of No. 1 single battery pack | Unit 0.001V Range: 0 ~ 65534 ( numeric offset 32767 , indicating -327.67 V ~ +327.67 V ) Description: (upload value - 32767)/1000V |
|  | ... | WORD | ... | Unit 0.001V Range: 0 ~ 65534 ( numeric offset 32767 , indicating -327.67 V ~ +327.67 V ) Description: (upload value - 32767)/1000V |
|  | N | WORD | Voltage of No. N single battery pack | Unit 0.001V Range: 0 ~ 65534 ( numeric offset 32767 , indicating -327.67 V ~ +327.67 V ) Description: (upload value - 32767)/1000V |

Note: Only the battery packs with the battery pack voltage mask set will be followed by the battery pack voltage byte

Example: 0x80000000 0x00000000 0x00000000 0x00000000 When it is a mask, it is followed by only the voltage of the first single battery pack .

Example: 0x88000000 0x00000000 0x00000000 0x00000000  When 0x8158 0x7EA6 is a mask, it is followed by the voltage of the No. 1 and No. 5 single-cell battery packs, as shown below:

No. 1 single battery pack is +3.45V ;

No. 5 single cell battery pack is -3.45V.

Appendix_New Energy Vehicle BMS Data Stream : Single Battery Pack Temperature Data Table

| Overall length | Byte sequence | type | content | describe |
| --- | --- | --- | --- | --- |
| 16+2*N | 0 | DWORD | Single battery pack temperature mask 0 | BIT31-BIT0 High bit first, low bit last BIT31: No. 1 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT30: No. 2 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 32 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 4 | DWORD | Cell pack temperature mask 1 | BIT31-BIT0 High bit first, low bit last BIT31: No. 33 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) .................. BIT0: No. 64 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 8 | DWORD | Cell pack temperature mask 2 | BIT31-BIT0 High bit first, low bit last BIT31: No. 65 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No. 96 single battery pack (if it is 1, there is a single battery pack byte, otherwise it is empty) |
|  | 12 | DWORD | Cell pack temperature mask 3 | BIT31-BIT0 High bit first, low bit last BIT31: No. 96 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT0: No. 128 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 16 | DWORD | Cell pack temperature mask 4 | BIT31-BIT0 High bit first, low bit last BIT31: No.129 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) .................. BIT0: No.160 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 20 | DWORD | Cell pack temperature mask 5 | BIT31-BIT0 High bit first, low bit last BIT31: No.161 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.192 single battery pack (if it is 1, there is a single battery pack byte, otherwise it is empty) |
|  | 24 | DWORD | Cell pack temperature mask 6 | BIT31-BIT0 High bit first, low bit last BIT31: No.193 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT0: No.224 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 28 | DWORD | Cell pack temperature mask 7 | BIT31-BIT0 High bit first, low bit last BIT31: No.225 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) .................. BIT0: No.256 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 32 | DWORD | Cell pack temperature mask 8 | BIT31-BIT0 High bit first, low bit last BIT31: No.257 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.288 single battery pack (if it is 1, there is a single battery pack byte, otherwise it is empty) |
|  | 36 | DWORD | Cell pack temperature mask 9 | BIT31-BIT0 High bit first, low bit last BIT31: No.289 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT0: No.320 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 40 | DWORD | Cell pack temperature mask 10 | BIT31-BIT0 High bit first, low bit last BIT31: No.321 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) .................. BIT0: No.352 single battery pack (if it is 1, there is a single battery pack byte, otherwise it is empty) |
|  | 44 | DWORD | Cell pack temperature mask 11 | BIT31-BIT0 High bit first, low bit last BIT31: No.353 single battery group (if it is 1, there is a single battery group byte, otherwise it is empty) BIT0: No.384 single battery pack (if 1, there is a single battery pack byte, otherwise it is empty) |
|  | 48 | WORD | Temperature of battery pack No. 1 | Unit 0.1 ℃ Range: 0 -- 240 0 ( numeric offset 40 ℃, means -40 ℃ -- + 200 ℃ ) , minimum measurement unit 0.1 ℃ . Description: (upload value - 400)/10 ℃ |
|  | ... | WORD | ... | Unit 0.1 ℃ Range: 0 -- 240 0 ( numeric offset 40 ℃, means -40 ℃ -- + 200 ℃ ) , minimum measurement unit 0.1 ℃ . Description: (upload value - 400)/10 ℃ |
|  | N | WORD | Temperature of the Nth battery pack | Unit: 0.1 ℃ Range: 0 -- 240 0 ( numeric offset 40 ℃, means -40 ℃ -- + 200 ℃ ) , minimum measurement unit 0.1 ℃ . Description: (upload value - 400)/10 ℃ |

Note: Only the battery packs with the battery pack temperature mask set will be followed by the battery pack temperature byte

Example: 0x80000000 0x00000000 0x00000000 0x00000000 When it is a mask, it is followed by only the temperature of the first single battery pack .

Example: 0x88000000 0x00000000 0x00000000 0x00000000  When 0x0000 0x0960 is a mask, it is followed by the temperature of the first and fifth battery packs, as shown below:

No. 1 single battery pack is -40.0 ℃ ;

No. 5 single battery pack is +200.0 ℃ .

Appendix_WIFI data flow

| content | Number of bytes | Data Types | describe |
| --- | --- | --- | --- |
| Wifi number | 1 | byte | Number of Wifi hotspots |
| , | 1 | byte | English half- width comma separator |
| ecn[0] | n | string | Wifi hotspot encryption method, backup, fixed to "-" |
| , | 1 | byte | English half- width comma separator |
| ssid[0] | n | string | Wifi hotspot name, spare, fixed "-" |
| , | 1 | byte | English half- width comma separator |
| rssi[0] | 1 | byte | Wifi hotspot signal strength, unit: dBm |
| , | 1 | byte | English half- width comma separator |
| m ac [ 0 ] | n | string | Wifi hotspot signal MAC address, such as "1C:20:DB:8D:D7:80" |
| , | 1 | byte | English half- width comma separator |
| channel [ 0 ]​ | 1 | byte | The channels used by Wifi hotspots vary according to the range returned by each module. |
| ecn[1] | n | string | Wifi hotspot encryption method, backup, fixed to "-" |
| , | 1 | byte | English half- width comma separator |
| ssid[1] | n | string | Wifi hotspot name, spare, fixed "-" |
| , | 1 | byte | English half- width comma separator |
| rssi[1] | 1 | byte | Wifi hotspot signal strength, unit: dBm |
| , | 1 | byte | English half- width comma separator |
| m ac [ 1 ] | n | string | Wifi hotspot signal MAC address, such as "1C:20:DB:8D:D7:80" |
| , | 1 | byte | English half- width comma separator |
| channel [ 1 ]​ | 1 | byte | The channels used by Wifi hotspots vary according to the range returned by each module. |
| ...... |  |  | when wifi num is greater than 1, enc[1], ssid[1], rssi[1],mac[1],channel[1] comma separated The format is as shown in the gray part above, which accumulates information of multiple WiFi hotspots in sequence . |

Appendix_Extended peripheral data: High - speed rail tire pressure data table

| Overall length | Byte sequence | type | length | content | describe |
| --- | --- | --- | --- | --- | --- |
| 8*N | 0 | u8 | 1 | Sensor Location No. X | The first axis has 2 wheels, numbered 0x01, 0x02, and the other axes can be adapted to 2 wheels or 4 wheels. For example, when the second axis has two wheels, the numbers are 0x11, 0x12, and when the second axis has four wheels, the numbers are 0x10, 0x11, 0x12, 0x13; When the third axis has two wheels, the numbers are 0x21, 0x22; when the third axis has four wheels, the numbers are 0x20, 0x21, 0x22, 0x23; and so on; |
|  | 1 | u16 | 2 | Sensor No. X Pressure | Unit 1 Kpa |
|  | 3 | u8 | 1 | Sensor No. X Temperature | Unit 1℃ Display value is the uploaded value minus 55℃ |
|  | 4 | u16 | 2 | Sensor No. X voltage | Unit: V, accuracy: 0.01 |
|  | 6 | u8 | 1 | Y-axis acceleration of tire No. X | Unit: g, precision: 0.1 |
|  | 7 | u8 | 1 | Z-axis acceleration of tire No. X | Unit: g, precision: 0.1 |
|  | ... | ... | ... | ... | ... |
|  | N | u8 | 1 | Sensor location X+N | The first axis has 2 wheels, numbered 0x01, 0x02, and the other axes can be adapted to 2 wheels or 4 wheels. For example, when the second axis has two wheels, the numbers are 0x11, 0x12, and when the second axis has four wheels, the numbers are 0x10, 0x11, 0x12, 0x13; When the third axis has two wheels, the numbers are 0x21, 0x22; when the third axis has four wheels, the numbers are 0x20, 0x21, 0x22, 0x23; and so on; |
|  | N+1 | u16 | 2 | Sensor pressure No. X+N | Unit 1 Kpa |
|  | N+3 | u8 | 1 | Temperature of sensor No. X+N | Unit 1℃ Display value is the uploaded value minus 55℃ |
|  | N+4 | u16 | 2 | Sensor voltage No. X+N | Unit: V, accuracy: 0.01 |
|  | N+6 | u8 | 1 | Y-axis acceleration of tire No. X+N | Unit: g, precision: 0.1 |
|  | N+7 | u8 | 1 | Z-axis acceleration of tire No. X+N | Unit: g, precision: 0.1 |

Schedule_Input and Output Status Table

The first 4 bytes represent the input state (high/low level), and each 2 BITs represent an input state. For example, Bit0-Bit1 represents IN1 (00: 01 is not supported: high level 10: low level) 11 reserved); the last 4 bytes represent the output state, and each 2 BIT represents an output state (floating/low level), for example, Bit0-Bit1 represents OUT1 (00: 01 is not supported: dangling 10: low battery) Ping 11 reserved), the specific definition is as follows:

| byte position | Field | Number of bytes | Data type | Description |
| --- | --- | --- | --- | --- |
| 0 | Input state | 1 | u8 | Bit0Bit1 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN1 |
|  |  |  |  | Bit2Bit3 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN2 |
|  |  |  |  | Bit4Bit5 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN3 |
|  |  |  |  | Bit6Bit7 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN4 |
| 1 |  | 1 | U8 | Bit0Bit1 00:not support 01:high voltage 10:low voltage 11:Reserve IN5 |
|  |  |  |  | Bit2Bit3 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN6 |
|  |  |  |  | Bit4Bit5 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN7 |
|  |  |  |  | Bit6Bit7 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:Reserve IN8 |
| 2 | Output state | 1 | u8 | Bit0Bit1 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT1 |
|  |  |  |  | Bit2Bit3 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT2 |
|  |  |  |  | Bit4Bit5 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT3 |
|  |  |  |  | Bit6Bit7 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT4 |
| 3 |  | 1 | u8 | Bit0Bit1 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT5 |
|  |  |  |  | Bit2Bit3 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float OUT6 |
|  |  |  |  | Bit4Bit5 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float 5V_OUT1 |
|  |  |  |  | Bit6Bit7 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float 5V_OUT2 |
| 4 |  | 1 | U8 | Bit0Bit1 00:not support 01:high voltage |
|  |  |  |  | 10:low voltage 11:float 12V_OUT |

Schedule_Driving Licence Data Packets(Nbytes)

| Package Information item格式 |  |  |  |
| --- | --- | --- | --- |
| Field | Data type | Number of bytes | Description and Requirements |
| Dynamic Information ID | u16 | 2 | 0x0001~0xFFFF |
| Dynamic information length | u16 | 2 | 1-65535 |
| Dynamic information Content | u8 | N | See the table below for dynamic information content |

| Function ID field | Function ID | Length | Function | Unit | Describe |
| --- | --- | --- | --- | --- | --- |
| Underlying data items [0X0001-0XFFFF] | 0x9009 | N | Driver swipe card data pack( Track 1-3) |  | See 3.83 Track1- 3 information |
|  | 0x9010 | 1 | Login status | U8 | 1: log in 0: log out |
|  | 0x9011 | 1 | DLT allow Flg | U8 | 0x01：correct 0x00:incorrect |

Track 1-3 information※

| byte position | Function ID(1byte) | Length(1byte) | Function | Unit | Describe |
| --- | --- | --- | --- | --- | --- |
| 0 | 0x9901 | n | Driver name | string | Driver name |
|  | 0x9902 | 6 | Country | Ascll | Country |
|  | 0x9903 | 13 | ID Number | Ascll | ID Number |
|  | 0x9904 | 4 | Exp Date | Ascll | Exp Date |
|  | 0x9905 | 8 | Birthday | Ascll | Birthday |
|  | 0x9906 | n | Type of Driving licese | Ascll | Type of Driving licese |
|  | 0x9907 | 1 | Gender | Ascll | Gender |
|  | 0x9908 | n | License Number | A scll | License Number |
|  | 0x9909 | 5 | Issuing Branch | A scll | Issuing Branch |
|  | 0x990A | n | All Track form card | A scll | T1+T2+T3 |

Track 3 information

| byte position | Function ID(1byte) | Length(1byte) | Function | Unit | Describe |
| --- | --- | --- | --- | --- | --- |
| 0 | 0x9906 | 4 | Type of Driving licese | Ascll | Type of Driving licese |
|  | 0x9907 | 1 | Gender | Ascll | Gender |
|  | 0x9908 | 7 | License Number | Ascll | License Number |
|  | 0x9909 | 5 | Issuing Branch | Ascll | Issuing Branch |
|  | 0x9011 | 1 | Driver license allow | U8 | 0x01:allow 0x00: not allow |

Appendix - CAN Broadcast Data Stream (Standard 808-2013)

| 起始字节 | 字段 | 字节数 | 数据类型 | 描述 |
| --- | --- | --- | --- | --- |
| 0 | Number of data items | 2 | WORD | The number of CAN bus data items included, with a value greater than 0 |
| 2 | CAN bus data reception time | 5 | BCD[5] | The receiving time of the first data on the CAN bus, hh mm ss msms |
| 7 | CAN bus data items | -- | -- | CAN bus data item format table |

CAN bus data item format table (Standard 808-2013)

| 起始字节 | 字段 | 字节数 | 数据类型 | 描述 |
| --- | --- | --- | --- | --- |
| 0 | CAN ID | 4 | DWORD | Bit31 represents the CAN channel number, 0: CAN1, 1: CAN2; Bit30 represents frame type, 0: standard frame, 1: extended frame; Bit29 represents the data collection method, 0: raw data, 1: average value of the collection interval; Bit28-bit0 represents the CAN bus ID |
| 4 | CAN DATA | 8 | BYTE[8] | CAN data |
| -- | -- | -- | -- | -- |

Appendix 2: Sample

Example of escape function:

/************************************************************************************

* Function name: void JT_EscapeData (u16 InLen,u8 *InBuf,u16 *OutLen,u8 *OutBuf)

* Description: escape

* InBuf: Input data that needs to be escaped

* InLen: Input the length of the data that needs to be escaped

* OutBuf: Output data that needs to be escaped

* OutLen: Output requires escape data length

*******************************************************************************/

void JT_EscapeData (u16 InLen,u8 *InBuf, u16 *OutLen, u8 *OutBuf)

{

u16 i=0;

u16 Len=0;

//Escape

for(i=0;i<InLen;i++)

{

if(InBuf[i]==0x7E)

{

OutBuf[Len++]=0x7D;

OutBuf[Len++]=0x02;

}

else if(InBuf[i]==0x7D)

{

OutBuf[Len++]=0x7D;

OutBuf[Len++]=0x01;

}

else

{

OutBuf[Len++]=InBuf[i];

}

}

*OutLen=Len;

}

An example of a reverse semantic function:

/************************************************************************************

* Function name: void JT_UnEscapeData (u16 InLen,u8 *InBuf,u16 *OutLen,u8 *OutBuf)

* Description: Input the data to be deinterpreted, and the original data after deinterpretation can be output

* InBuf: Input data that needs to be unsequenced

* InLen: Input the length of the data to be unset

* OutBuf: Output the data that needs to be deinterlaced

* OutLen: Output data length that needs to be reversed

*******************************************************************************/

void JT_UnEscapeData (u16 InLen,u8 *InBuf,u16 *OutLen,u8 *OutBuf)

{

u16 i=0;

u16 ValidPos=0;

if(InBuf[0]!=0x7E)

return 0;

OutBuf[ValidPos++]=0x7E;

for(i=1;i<InLen;i++)

{

if(InBuf[i]==0x7D)

{

if(InBuf[i+1]==0x01)

{

OutBuf[ValidPos++]=0x7D;

i++;

}

else if(InBuf[i+1]==0x02)

{

OutBuf[ValidPos++]=0x7E;

i++;

}

else  return 0;

}

else

{

OutBuf[ValidPos++]=InBuf[i];

}

if(InBuf[i]==0x7E)

{

break;

}

}

if(i==InLen)

return 0;

*OutLen=ValidPos;

return (i+1);

}

[ 0200 ] Location data analysis details

[ 0900 ] Uplink data analysis details

[ 8300 ] Text information data analysis details